Contents

Basic Automation using Telnet (telnetlib)	2

Section 1 (telnetlib) - Script 1	4

Section 1 (telnetlib) - Script 2	6

Section 1 (telnetlib) - Script 3	8

Basic Automation using SSH (netmiko)	10

Netmiko basics	10

Netmiko Library	10

Section 2 (netmiko) - Script 1 - configuring a single switch	13

Section 2 (netmiko) - Script 2 - configuring multiple switches	15

Section 2 (netmiko) - Script 3 - configuring multiple switches using a file config	18

Section 2 (netmiko) - Script 4 - configuring multiple switches using a 2 files with config	20

Network Automation with NAPALM	24

Section 3 (napalm) - Script 1 - Get remote device info using NAPALM	27

Section 3 (napalm) - Script 2 - Get remote device info using NAPALM (improved output)	28

Section 3 (napalm) - Script 3 - Get remote device info using NAPALM (improved output using JSON)	29

Section 3 (napalm) - Script 4 - Get more remote device info using NAPALM (improved output using JSON)	31

Section 3 (napalm) - Script 5 - Get BGP neighbor info using NAPALM  and print the output using JSON	34

Section 3 (napalm) - Script 6 - Making config changes to remote devices using NAPALM	36

Section 3 (napalm) - Script 6 - Check for diff and make config changes to remote devices using NAPALM	38

Section 3 (napalm) - Script 7 - Check for config diff multiple devices using NAPALM	40

Section 8 - Iterations with Netmiko	42

Section 8 (iterations with Netmiko) - Iteration 1 - Send a 'show' command	42

Section 8 (iterations with Netmiko) - Iteration 2 - Send commands from a file	44

Section 8 (iterations with Netmiko) - Iteration 3 - Send commands from a file to multiple devices	46

Section 8 (iterations with Netmiko) - Iteration 4 - Send commands from a file to multiple devices with user prompt	48

Section 8 (iterations with Netmiko) - Iteration 5 - Send commands from a file to multiple devices with user prompt and error handling	50

Section 9 - Scaling Python scripts	53

Section 9 (scaling Netmiko) - Encrypt a passwords file using simple-crypt	53

Section 9 (scaling Netmiko) - Measuring the script execution time in Netmiko sequential deployment	55

## Basic Automation using Telnet (telnetlib)

Topology

Network Automation-1 pc DHCP config:

Step 1

Change the following:

root@NetworkAutomation-1:/etc/network# cat interfaces

#

# This is a sample network config uncomment lines to configure the network

#

# Static config for eth0

#auto eth0

#iface eth0 inet static

#       address 192.168.0.2

#       netmask 255.255.255.0

#       gateway 192.168.0.1

#       up echo nameserver 192.168.0.1 > /etc/resolv.conf

# DHCP config for eth0

# auto eth0

# iface eth0 inet dhcp

to:

root@NetworkAutomation-1:/etc/network# cat interfaces

#

# This is a sample network config uncomment lines to configure the network

#

# Static config for eth0

#auto eth0

#iface eth0 inet static

#       address 192.168.0.2

#       netmask 255.255.255.0

#       gateway 192.168.0.1

#       up echo nameserver 192.168.0.1 > /etc/resolv.conf

# DHCP config for eth0

auto eth0

iface eth0 inet dhcp

Step 2

Reboot the VM (right-click from GNS3)

Verification

root@NetworkAutomation-1:~# ifconfig

eth0      Link encap:Ethernet  HWaddr d6:17:0f:b9:04:21

inet addr:192.168.122.225  Bcast:192.168.122.255  Mask:255.255.255.0

UP BROADCAST RUNNING MULTICAST  MTU:1500  Metric:1

RX packets:170875 errors:0 dropped:53 overruns:0 frame:0

TX packets:168894 errors:0 dropped:0 overruns:0 carrier:0

collisions:0 txqueuelen:1000

RX bytes:7197250 (7.1 MB)  TX bytes:7097415 (7.0 MB)

lo        Link encap:Local Loopback

inet addr:127.0.0.1  Mask:255.0.0.0

inet6 addr: ::1/128 Scope:Host

UP LOOPBACK RUNNING  MTU:65536  Metric:1

RX packets:0 errors:0 dropped:0 overruns:0 frame:0

TX packets:0 errors:0 dropped:0 overruns:0 carrier:0

collisions:0 txqueuelen:1

RX bytes:0 (0.0 B)  TX bytes:0 (0.0 B)

Switch and Router configuration

Here is a sample config that allows Telnet reachability:

SW2(config)#int range e0/0 - 3 , e1/0

SW2(config-if-range)#switchport trunk encapsulation dot1q

SW2(config-if-range)#switchport mode trunk

SW2(config-if-range)#!

SW2(config-if-range)#int vlan 1

SW2(config-if)#no shut

SW2(config-if)#ip add 192.168.122.82 255.255.255.0

SW2(config-if)#!

SW2(config-if)#line vty 0 4

SW2(config-line)#login local

SW2(config-line)#transport input all

SW2(config-line)#exit

SW2(config)#username cisco password cisco

SW2(config)#username mikis password cisco

SW2(config)#enable password cisco

Verification

root@NetworkAutomation-1:~# telnet 192.168.122.81

Trying 192.168.122.81...

Connected to 192.168.122.81.

Escape character is '^]'.

User Access Verification

Username: cisco

Password:

SW1>enable

Password:

SW1#

### Section 1 (telnetlib) - Script 1

The following script:

- Connects via telnet to a device

- The IP of the device is specified manually (the HOST variable)

- The user is prompted for user and password

- The provided credentials are used by the script in the telnet connection

- The script creates 2 loopback interfaces

import getpass

import telnetlib

# variable with the device IP

HOST = "192.168.122.71"

# prompt the user and save the input to a variable

user = input("Enter your telnet username: ")

# from 'getpass' module use getpass parameter to save the password to a variable

password = getpass.getpass()

# from 'telnetlib' library use Telnet class to telnet to an IP

# save what is returned to a variable

tn = telnetlib.Telnet(HOST)

# read the variable until you see 'Username'

tn.read_until(b"Username: ")

# send the user input (username in this case) in ASCII format

# along with a new line to the device

tn.write(user.encode('ascii') + b"\n")

# if user provided a password:

# find the string 'Password'

# send the provided password in ASCII format to the device

if password:

tn.read_until(b"Password: ")

tn.write(password.encode('ascii') + b"\n")

# send 'enable' to the device

tn.write(b"enable\n")

# send the following commands to the device

tn.write(b"cisco\n")

tn.write(b"conf t\n")

tn.write(b"int loop 0\n")

tn.write(b"ip address 1.1.1.1 255.255.255.255\n")

tn.write(b"int loop 1\n")

tn.write(b"ip address 2.2.2.2 255.255.255.255\n")

tn.write(b"end\n")

tn.write(b"wr\n")

tn.write(b"exit\n")

# print what was read by the device and decode it in ASCII

print(tn.read_all().decode('ascii'))

### Section 1 (telnetlib) - Script 2

The following script:

- Connects via telnet to a device

- The IP of the device is specified manually (the HOST variable)

- The user is prompted for user and password

- The provided credentials are used by the script in the telnet connection

- The script uses an if loop to create 10 VLANs

import getpass

import telnetlib

# Variable with the device IP

HOST = "192.168.122.72"

# Prompt the user and save the input to a variable

user = input("Enter your telnet username: ")

# From 'getpass' module use getpass parameter to save the password to a variable

password = getpass.getpass()

# From 'telnetlib' library use Telnet class to telnet to an IP

# Save what is returned to a variable named 'tn'

tn = telnetlib.Telnet(HOST)

# Read the variable until you see 'Username'

tn.read_until(b"Username: ")

# Send the user input (username in this case) in ASCII format

# Along with a new line to the device

tn.write(user.encode('ascii') + b"\n")

# If user provided a password:

# Find the string 'Password'

# Send the provided password in ASCII format to the device

if password:

tn.read_until(b"Password: ")

tn.write(password.encode('ascii') + b"\n")

# Send 'enable' to the device

tn.write(b"enable\n")

# Send the following commands to the device

tn.write(b"cisco\n")

tn.write(b"conf t\n")

# Loop for the creation of 10 VLANs

for n in range (2,11):

tn.write(b"vlan " + str(n).encode('ascii') + b"\n")

tn.write(b"name Python_VLAN_" + str(n).encode('ascii') + b"\n")

tn.write(b"end\n")

tn.write(b"wr\n")

tn.write(b"exit\n")

# Print what was read by the device and decode it in ASCII

print(tn.read_all().decode('ascii'))

### Section 1 (telnetlib) - Script 3

The following script:

- Connects via telnet to multiple devices

- The IP of the device are read by the script from a file

- The user is prompted for user and password

- The provided credentials are used by the script in the telnet connection

- The script saves the running config of the remote devices to separate files

# This script gets the running config from multiple devices

# The IP of the devices are within a file

# Import the getpass and telnetlib modules

import getpass

import telnetlib

# Get an input from the user and pass it to a variable

user = input("Enter your username: ")

# Get an input from the user

# the default prompt for 'getpass' is 'Password: '

password = getpass.getpass()

# Open a file. This file contains the IPs of the devices

f = open('myswitches.txt')

# For loop that reads the opened file

for IP in f:

# Strip whitespace characters from beginning and end of IP object

# Add the result to a variable

IP=IP.strip()

# Print a line and concatenate

print ("Get running config from Switch " + (IP))

# Put the IP to a variable

HOST = IP

# Telnet to the device

tn = telnetlib.Telnet(HOST)

# Read in the telnet session until you find the 'Username: ' prompt

# As soon as you find it put the username you got before

tn.read_until(b"Username: ")

tn.write(user.encode('ascii') + b"\n")

# The following 'if' statement returns true when the 'password' value

# is not equal to 0 (when x is a number)

# and it returns true if it has at least a character(when x is a string)

# In this case if the user has entered a password

# then read until you find the 'password' prompt from telnet

# and enter the password provided by the user before

if password:

tn.read_until(b"Password: ")

tn.write(password.encode('ascii') + b"\n")

# Runs a series of IOS commands and exit the telnet session

tn.write(b"enable\n")

tn.write(b"cisco\n")

tn.write(b"terminal length 0\n")

tn.write(b"show run\n")

tn.write(b"exit\n")

# Read until EOF and save the output to a variable

read = tn.read_all()

# open a file for writing

# The 'w' option allows the file creation if it doesn't exist

save = open("switch" + HOST, "w")

# Write to the file and add an empty line

# Finally, close the file and start the 'if' loop again

save.write(read.decode('ascii'))

save.write("\n")

save.close

## Basic Automation using SSH (netmiko)

Topology

Step 1

Prepare the devices for SSH access

SW1(config)#hostname SW1

SW1(config)#ip domain-name mikis.lab

SW1(config)#crypto key generate rsa

The name for the keys will be: SW1.mikis.lab

Choose the size of the key modulus in the range of 360 to 4096 for your

General Purpose Keys. Choosing a key modulus greater than 512 may take

a few minutes.

How many bits in the modulus [512]: 1024

% Generating 1024 bit RSA keys, keys will be non-exportable...

[OK] (elapsed time was 0 seconds)

SW1(config)#line vty 0 4

SW1(config-line)#transport input ssh

SW1(config-line)#login local

SW1(config)#username cisco password cisco

SW1(config)#username cisco privilege 15

SW1(config)#enable secret cisco

Verification

root@NetworkAutomation-1:~# ssh -l cisco 192.168.122.81

Bad owner or permissions on /root/.ssh/config

root@NetworkAutomation-1:~# chmod 600 /root/.ssh/config

root@NetworkAutomation-1:~# chown -R root /root/.ssh/config

root@NetworkAutomation-1:~# ssh -l cisco 192.168.122.81

Password:

SW1>enable

Password:

SW1#

### Netmiko basics

From https://pynet.twb-tech.com/blog/automation/netmiko.html:

### Netmiko Library

###### By Kirk Byers

###### 2015-06-19

Since late 2014, I have been working on an open-source Python library that simplifies SSH management to network devices. The library is based on the Paramiko SSH library and is named Netmiko.

You can find the library at https://github.com/ktbyers/netmiko and the latest released version of the software can be downloaded here.

The purposes of the library are the following:

- Successfully establish an SSH connection to the device

- Simplify the execution of show commands and the retrieval of output data

- Simplify execution of configuration commands including possibly commit actions

- Do the above across a broad set of networking vendors and platforms

I have observed across time that many people run into similar issues managing Python SSH sessions to network devices. For example, HP ProCurve switches have ANSI escape codes in the output or the Cisco WLC has an extra 'login as:' message. These types of issues can soak up a lot of development and troubleshooting time and what is worse people keep solving the same issues over and over again (including sometimes not solving them and giving up).

So Netmiko intends to simplify this lower-level SSH management across a wide set of networking vendors and platforms.

As of June 2015, Netmiko has support for the following platforms:

- Cisco IOS

- Cisco IOS-XE

- Cisco ASA

- Cisco NX-OS

- Cisco IOS-XR

- Cisco WLC (limited testing)

- Arista EOS

- HP ProCurve

- HP Comware (limited testing)

- Juniper Junos

- Brocade VDX (limited testing)

- F5 LTM (experimental)

- Huawei (limited testing)

As with anything involving programming, test extensively in your own environment and for your own specific use cases. It is your responsibility to know what you are doing and to have good testing processes in place.

If I try to run a python program that uses Netmiko I get an error:

root@NetworkAutomation-1:~# python3 netmiko1.py

Traceback (most recent call last):

File "netmiko1.py", line 2, in <module>

from netmiko import ConnectHandler

ImportError: No module named 'netmiko'

To fix the above I have to do the following:

root@NetworkAutomation-1:~# apt-get update

Get:1 http://security.ubuntu.com/ubuntu xenial-security InRelease [107 kB]

Get:2 http://archive.ubuntu.com/ubuntu xenial InRelease [247 kB]

Get:3 http://ppa.launchpad.net/ansible/ansible-2.5/ubuntu xenial InRelease [18.0 kB]

Get:4 http://archive.ubuntu.com/ubuntu xenial-updates InRelease [109 kB]

Get:5 http://archive.ubuntu.com/ubuntu xenial-backports InRelease [107 kB]

Get:6 http://ppa.launchpad.net/ansible/ansible-2.5/ubuntu xenial/main amd64 Packages [469 B]

Get:7 http://security.ubuntu.com/ubuntu xenial-security/universe Sources [90.3 kB]

Get:8 http://security.ubuntu.com/ubuntu xenial-security/main amd64 Packages [703 kB]

Get:9 http://archive.ubuntu.com/ubuntu xenial/universe Sources [9802 kB]

Get:10 http://security.ubuntu.com/ubuntu xenial-security/restricted amd64 Packages [12.7 kB]

Get:11 http://security.ubuntu.com/ubuntu xenial-security/universe amd64 Packages [468 kB]

Get:12 http://security.ubuntu.com/ubuntu xenial-security/multiverse amd64 Packages [3748 B]

Get:13 http://archive.ubuntu.com/ubuntu xenial/main amd64 Packages [1558 kB]

Get:14 http://archive.ubuntu.com/ubuntu xenial/restricted amd64 Packages [14.1 kB]

Get:15 http://archive.ubuntu.com/ubuntu xenial/universe amd64 Packages [9827 kB]

Get:16 http://archive.ubuntu.com/ubuntu xenial/multiverse amd64 Packages [176 kB]

Get:17 http://archive.ubuntu.com/ubuntu xenial-updates/universe Sources [277 kB]

Get:18 http://archive.ubuntu.com/ubuntu xenial-updates/main amd64 Packages [1088 kB]

Get:19 http://archive.ubuntu.com/ubuntu xenial-updates/restricted amd64 Packages [13.1 kB]

Get:20 http://archive.ubuntu.com/ubuntu xenial-updates/universe amd64 Packages [878 kB]

Get:21 http://archive.ubuntu.com/ubuntu xenial-updates/multiverse amd64 Packages [18.8 kB]

Get:22 http://archive.ubuntu.com/ubuntu xenial-backports/main amd64 Packages [7343 B]

Get:23 http://archive.ubuntu.com/ubuntu xenial-backports/universe amd64 Packages [8086 B]

Fetched 25.5 MB in 30s (850 kB/s)

Reading package lists... Done

root@NetworkAutomation-1:~# apt-get install python3-pip

Reading package lists... Done

Building dependency tree

Reading state information... Done

python3-pip is already the newest version (8.1.1-2ubuntu0.4).

0 upgraded, 0 newly installed, 0 to remove and 55 not upgraded.

root@NetworkAutomation-1:~# apt-get install python3-dev

root@NetworkAutomation-1:~# pip3 install -U netmiko

Collecting netmiko

Using cached https://files.pythonhosted.org/packages/1c/82/9aef0c1b4fca782fd18bee44c9ed18404060f1c04ecbf1386594f5f41834/netmiko-2.2.2.tar.gz

Collecting paramiko>=2.0.0 (from netmiko)

Using cached https://files.pythonhosted.org/packages/3e/db/cb7b6656e0e7387637ce850689084dc0b94b44df31cc52e5fc5c2c4fd2c1/paramiko-2.4.1-py2.py3-none-any.whl

Collecting scp>=0.10.0 (from netmiko)

Using cached https://files.pythonhosted.org/packages/ef/8c/399c5ddffc8c1bd2593ee6c8937ed31f005dcfa0c591da149f7b4b993f14/scp-0.11.0-py2.py3-none-any.whl

Collecting pyyaml (from netmiko)

Using cached https://files.pythonhosted.org/packages/9e/a3/1d13970c3f36777c583f136c136f804d70f500168edc1edea6daa7200769/PyYAML-3.13.tar.gz

Collecting pyserial (from netmiko)

Using cached https://files.pythonhosted.org/packages/0d/e4/2a744dd9e3be04a0c0907414e2a01a7c88bb3915cbe3c8cc06e209f59c30/pyserial-3.4-py2.py3-none-any.whl

Collecting textfsm (from netmiko)

Using cached https://files.pythonhosted.org/packages/a1/0d/a1b490503545b3b4600b965eae5d44cc2b6ce27cfb44f4debc563dbb56d3/textfsm-0.4.1.tar.gz

Requirement already up-to-date: pynacl>=1.0.1 in /usr/local/lib/python3.5/dist-packages (from paramiko>=2.0.0->netmiko)

Collecting cryptography>=1.5 (from paramiko>=2.0.0->netmiko)

Using cached https://files.pythonhosted.org/packages/22/21/233e38f74188db94e8451ef6385754a98f3cad9b59bedf3a8e8b14988be4/cryptography-2.3.1.tar.gz

Requirement already up-to-date: bcrypt>=3.1.3 in /usr/local/lib/python3.5/dist-packages (from paramiko>=2.0.0->netmiko)

Requirement already up-to-date: pyasn1>=0.1.7 in /usr/local/lib/python3.5/dist-packages (from paramiko>=2.0.0->netmiko)

Requirement already up-to-date: six in /usr/local/lib/python3.5/dist-packages (from pynacl>=1.0.1->paramiko>=2.0.0->netmiko)

Requirement already up-to-date: cffi>=1.4.1 in /usr/local/lib/python3.5/dist-packages (from pynacl>=1.0.1->paramiko>=2.0.0->netmiko)

Requirement already up-to-date: idna>=2.1 in /usr/local/lib/python3.5/dist-packages (from cryptography>=1.5->paramiko>=2.0.0->netmiko)

Requirement already up-to-date: asn1crypto>=0.21.0 in /usr/local/lib/python3.5/dist-packages (from cryptography>=1.5->paramiko>=2.0.0->netmiko)

Requirement already up-to-date: pycparser in /usr/local/lib/python3.5/dist-packages (from cffi>=1.4.1->pynacl>=1.0.1->paramiko>=2.0.0->netmiko)

Building wheels for collected packages: netmiko, pyyaml, textfsm, cryptography

Running setup.py bdist_wheel for netmiko ... error

Complete output from command /usr/bin/python3 -u -c "import setuptools, tokenize;__file__='/tmp/pip-build-ajv85x1q/netmiko/setup.py';exec(compile(getattr(tokenize, 'open', open)(__file__).read().replace('\r\n', '\n'), __file__, 'exec'))" bdist_wheel -d /tmp/tmpx8aep4i7pip-wheel- --python-tag cp35:

/usr/lib/python3.5/distutils/dist.py:261: UserWarning: Unknown distribution option: 'long_description_content_type'

warnings.warn(msg)

usage: -c [global_opts] cmd1 [cmd1_opts] [cmd2 [cmd2_opts] ...]

or: -c --help [cmd1 cmd2 ...]

or: -c --help-commands

or: -c cmd --help

error: invalid command 'bdist_wheel'

----------------------------------------

Failed building wheel for netmiko

Running setup.py clean for netmiko

Running setup.py bdist_wheel for pyyaml ... error

Complete output from command /usr/bin/python3 -u -c "import setuptools, tokenize;__file__='/tmp/pip-build-ajv85x1q/pyyaml/setup.py';exec(compile(getattr(tokenize, 'open', open)(__file__).read().replace('\r\n', '\n'), __file__, 'exec'))" bdist_wheel -d /tmp/tmp8tt0aplmpip-wheel- --python-tag cp35:

usage: -c [global_opts] cmd1 [cmd1_opts] [cmd2 [cmd2_opts] ...]

or: -c --help [cmd1 cmd2 ...]

or: -c --help-commands

or: -c cmd --help

error: invalid command 'bdist_wheel'

----------------------------------------

Failed building wheel for pyyaml

Running setup.py clean for pyyaml

Running setup.py bdist_wheel for textfsm ... error

Complete output from command /usr/bin/python3 -u -c "import setuptools, tokenize;__file__='/tmp/pip-build-ajv85x1q/textfsm/setup.py';exec(compile(getattr(tokenize, 'open', open)(__file__).read().replace('\r\n', '\n'), __file__, 'exec'))" bdist_wheel -d /tmp/tmp34iogmyupip-wheel- --python-tag cp35:

/usr/lib/python3.5/distutils/dist.py:261: UserWarning: Unknown distribution option: 'long_description_content_type'

warnings.warn(msg)

usage: -c [global_opts] cmd1 [cmd1_opts] [cmd2 [cmd2_opts] ...]

or: -c --help [cmd1 cmd2 ...]

or: -c --help-commands

or: -c cmd --help

error: invalid command 'bdist_wheel'

----------------------------------------

Failed building wheel for textfsm

Running setup.py clean for textfsm

Running setup.py bdist_wheel for cryptography ... error

Complete output from command /usr/bin/python3 -u -c "import setuptools, tokenize;__file__='/tmp/pip-build-ajv85x1q/cryptography/setup.py';exec(compile(getattr(tokenize, 'open', open)(__file__).read().replace('\r\n', '\n'), __file__, 'exec'))" bdist_wheel -d /tmp/tmphhryp5q9pip-wheel- --python-tag cp35:

/usr/lib/python3.5/distutils/dist.py:261: UserWarning: Unknown distribution option: 'python_requires'

warnings.warn(msg)

usage: -c [global_opts] cmd1 [cmd1_opts] [cmd2 [cmd2_opts] ...]

or: -c --help [cmd1 cmd2 ...]

or: -c --help-commands

or: -c cmd --help

error: invalid command 'bdist_wheel'

----------------------------------------

Failed building wheel for cryptography

Running setup.py clean for cryptography

Failed to build netmiko pyyaml textfsm cryptography

Installing collected packages: cryptography, paramiko, scp, pyyaml, pyserial, textfsm, netmiko

Running setup.py install for cryptography ... /                                                            done

Running setup.py install for pyyaml ... done

Running setup.py install for textfsm ... done

Running setup.py install for netmiko ... done

Successfully installed cryptography-2.3.1 netmiko-2.2.2 paramiko-2.4.1 pyserial-3.4 pyyaml-3.13 scp-0.11.0 textfsm-0.4.1

You are using pip version 8.1.1, however version 18.0 is available.

You should consider upgrading via the 'pip install --upgrade pip' command.

### Section 2 (netmiko) - Script 1 - configuring a single switch

The following script:

- Connects to one device via SSH

- The IP of the device is manually specified in the script

- It runs a 'show' command

- It runs few 'config' comands

# From netmiko module import ConnectHandler

from netmiko import ConnectHandler

# Create a variable that contains a python Dictionary with 4 keys

iosv_l2 = {

'device_type': 'cisco_ios',

'ip': '192.168.122.72',

'username': 'cisco',

'password': 'cisco',

}

# The ** means that the argument is a dictionary

# One * would mean that the argument is a list

# The ConnectHandler will SSH to the device

net_connect = ConnectHandler(**iosv_l2)

# To view something from the remote device over SSH

# Netmiko 'send_command' method to send a 'show' command to the remote device and print the output

output = net_connect.send_command('show ip int brief')

print (output)

# To configure something to the remote device over SSH:

# A variable that contains a list

# Send the 2 commands and print the output

# 'send_config_set' is a Netmiko method to send configuration commands

config_commands = ['int loop 0', 'ip address 1.1.1.1 255.255.255.255']

output = net_connect.send_config_set(config_commands)

print (output)

# Here I use a loop to create 2 VLANs

# The Netmiko 'send_config_set' method sends the commands over SSH:

for n in range (2,6):

print ("Creating VLAN " + str(n))

config_commands = ['vlan ' + str(n), 'name Python_VLAN ' + str(n)]

output = net_connect.send_config_set(config_commands)

print (output)

Running the script

root@NetworkAutomation-1:~# python3 netmiko1.py

Interface              IP-Address      OK? Method Status                Protocol

Ethernet0/0            unassigned      YES unset  up                    up

Ethernet0/1            unassigned      YES unset  up                    up

Ethernet0/2            unassigned      YES unset  up                    up

Ethernet0/3            unassigned      YES unset  up                    up

Ethernet1/0            unassigned      YES unset  up                    up

Ethernet1/1            unassigned      YES unset  up                    up

Ethernet1/2            unassigned      YES unset  up                    up

Ethernet1/3            unassigned      YES unset  up                    up

...

Loopback0              1.1.1.1         YES manual up                    up

Vlan1                  192.168.122.72  YES NVRAM  up                    up

config term

Enter configuration commands, one per line.  End with CNTL/Z.

SW1(config)#int loop 0

SW1(config-if)#ip address 1.1.1.1 255.255.255.255

SW1(config-if)#end

SW1#

Creating VLAN 2

config term

Enter configuration commands, one per line.  End with CNTL/Z.

SW1(config)#vlan 2

SW1(config-vlan)#name Python_VLAN 2

SW1(config-vlan)#end

SW1#

Creating VLAN 3

config term

Enter configuration commands, one per line.  End with CNTL/Z.

SW1(config)#vlan 3

SW1(config-vlan)#name Python_VLAN 3

SW1(config-vlan)#end

SW1#

Creating VLAN 4

config term

Enter configuration commands, one per line.  End with CNTL/Z.

SW1(config)#vlan 4

SW1(config-vlan)#name Python_VLAN 4

SW1(config-vlan)#end

SW1#

Creating VLAN 5

config term

Enter configuration commands, one per line.  End with CNTL/Z.

SW1(config)#vlan 5

SW1(config-vlan)#name Python_VLAN 5

SW1(config-vlan)#end

SW1#

root@NetworkAutomation-1:~#

Verification

SW1#show vlan brief

VLAN Name                             Status    Ports

---- -------------------------------- --------- -------------------------------

1    default                          active    Et0/0, Et0/1, Et0/2, Et0/3

Et1/2, Et1/3, Et2/0, Et2/1

Et2/2, Et2/3, Et3/0, Et3/1

Et3/2, Et3/3

2    Python_VLAN 2                    active

3    Python_VLAN 3                    active

4    Python_VLAN 4                    active

5    Python_VLAN 5                    active

### Section 2 (netmiko) - Script 2 - configuring multiple switches

The following script:

- Connects to multiple devices via SSH

- The IP of the devices is manually specified in the script

- It runs few 'config' comands

from netmiko import ConnectHandler

# Create one variable for each remove device

# using a Python Dictionary for each device

ios_l2_sw1 = {

'device_type': 'cisco_ios',

'ip': '192.168.122.72',

'username': 'cisco',

'password': 'cisco'

}

ios_l2_sw2 = {

'device_type': 'cisco_ios',

'ip': '192.168.122.82',

'username': 'cisco',

'password': 'cisco'

}

ios_l2_sw3 = {

'device_type': 'cisco_ios',

'ip': '192.168.122.83',

'username': 'cisco',

'password': 'cisco'

}

# Create a list containing all the device variables

all_devices = [ios_l2_sw1, ios_l2_sw2, ios_l2_sw3]

# 'for' loop which connects to each device

for device in all_devices:

net_connect = ConnectHandler(**device)

# 'for' loop which creates 2 VLANs on each device

for n in range (2,4):

# Print a string with the VLAN number

print ("Creating VLAN " + str(n))

# Send config commands over SSH. The commands are in a list

config_commands = ['vlan ' + str(n), 'name Python_VLAN ' + str(n)]

output = net_connect.send_config_set(config_commands)

# Print the commands

print (output)

Running the script

root@NetworkAutomation-1:~# python3 netmiko2.py

Creating VLAN 2

config term

Enter configuration commands, one per line.  End with CNTL/Z.

SW1(config)#vlan 2

SW1(config-vlan)#name Python_VLAN 2

SW1(config-vlan)#end

SW1#

Creating VLAN 3

config term

Enter configuration commands, one per line.  End with CNTL/Z.

SW1(config)#vlan 3

SW1(config-vlan)#name Python_VLAN 3

SW1(config-vlan)#end

SW1#

Creating VLAN 2

config term

Enter configuration commands, one per line.  End with CNTL/Z.

SW2(config)#vlan 2

SW2(config-vlan)#name Python_VLAN 2

SW2(config-vlan)#end

SW2#

Creating VLAN 3

config term

Enter configuration commands, one per line.  End with CNTL/Z.

SW2(config)#vlan 3

SW2(config-vlan)#name Python_VLAN 3

SW2(config-vlan)#end

SW2#

Creating VLAN 2

config term

Enter configuration commands, one per line.  End with CNTL/Z.

SW3(config)#vlan 2

SW3(config-vlan)#name Python_VLAN 2

SW3(config-vlan)#end

SW3#

Creating VLAN 3

config term

Enter configuration commands, one per line.  End with CNTL/Z.

SW3(config)#vlan 3

SW3(config-vlan)#name Python_VLAN 3

SW3(config-vlan)#end

SW3#

root@NetworkAutomation-1:~#

Verification

When the script is not connected to the device:

SW3#show ssh

%No SSHv2 server connections running.

%No SSHv1 server connections running.

When it is connected:

SW3#show ssh

Connection Version Mode Encryption  Hmac         State                 Username

0          2.0     IN   aes128-cbc  hmac-sha1    Session started       cisco

0          2.0     OUT  aes128-cbc  hmac-sha1    Session started       cisco

SW3#show vlan brief

VLAN Name                             Status    Ports

---- -------------------------------- --------- -------------------------------

1    default                          active    Et0/0, Et0/1, Et0/2, Et0/3

Et1/0, Et1/2, Et1/3, Et2/0

Et2/1, Et2/2, Et2/3, Et3/0

Et3/1, Et3/2, Et3/3

2    Python_VLAN 2                    active

3    Python_VLAN 3                    active

1002 fddi-default                     act/unsup

1003 token-ring-default               act/unsup

1004 fddinet-default                  act/unsup

1005 trnet-default                    act/unsup

### Section 2 (netmiko) - Script 3 - configuring multiple switches using a file config

The following script:

- Connects to multiple devices via SSH

- The IP of the devices is manually specified in the script

- It reads a file which contains configuration and applies it

from netmiko import ConnectHandler

# 3 variables for 3 remote devices

# One Python Dictionary per remote device

ios_l2_sw4 = {

'device_type': 'cisco_ios',

'ip': '192.168.122.84',

'username': 'cisco',

'password': 'cisco'

}

ios_l2_sw5 = {

'device_type': 'cisco_ios',

'ip': '192.168.122.85',

'username': 'cisco',

'password': 'cisco'

}

ios_l2_sw6 = {

'device_type': 'cisco_ios',

'ip': '192.168.122.86',

'username': 'cisco',

'password': 'cisco'

}

# Read a file and split its lines

# The 'print' command is optional, but provides visibility

with open('switch_configs.txt') as f:

lines = f.read().splitlines()

print (lines)

# A list of the 3 devices that we want to connect

all_devices = [ios_l2_sw4, ios_l2_sw5, ios_l2_sw6]

# A 'for' loop which connects to the remote devices

# I sents the configuration read from the file

# The 'print' will show us what is applied

for devices in all_devices:

net_connect = ConnectHandler(**devices)

output = net_connect.send_config_set(lines)

print (output)

Running the script

root@NetworkAutomation-1:~# python3 netmiko3.py

['vlan 55', ' name VLAN_55', '!', 'router ospf 1', ' network 0.0.0.0 255.255.255.255 area 0 ', '!']

config term

Enter configuration commands, one per line.  End with CNTL/Z.

SW4(config)#vlan 55

SW4(config-vlan)# name VLAN_55

SW4(config-vlan)#!

SW4(config-vlan)#router ospf 1

SW4(config-router)# network 0.0.0.0 255.255.255.255 area 0

SW4(config-router)#!

SW4(config-router)#end

SW4#

config term

Enter configuration commands, one per line.  End with CNTL/Z.

SW5(config)#vlan 55

SW5(config-vlan)# name VLAN_55

SW5(config-vlan)#!

SW5(config-vlan)#router ospf 1

SW5(config-router)# network 0.0.0.0 255.255.255.255 area 0

SW5(config-router)#!

SW5(config-router)#end

SW5#

config term

Enter configuration commands, one per line.  End with CNTL/Z.

SW6(config)#vlan 55

SW6(config-vlan)# name VLAN_55

SW6(config-vlan)#!

SW6(config-vlan)#router ospf 1

SW6(config-router)# network 0.0.0.0 255.255.255.255 area 0

SW6(config-router)#!

SW6(config-router)#end

SW6#

root@NetworkAutomation-1:~#

Verification

SW4#sh run | s router

router ospf 1

network 0.0.0.0 255.255.255.255 area 0

SW4#sh ip ospf int br

Interface    PID   Area            IP Address/Mask    Cost  State Nbrs F/C

Vl1          1     0               192.168.122.84/24  1     DROTH 0/2

SW4#

*Sep 10 18:02:51.524: %OSPF-5-ADJCHG: Process 1, Nbr 192.168.122.86 on Vlan1 from LOADING to FULL, Loading Done

*Sep 10 18:02:59.016: %OSPF-5-ADJCHG: Process 1, Nbr 192.168.122.85 on Vlan1 from LOADING to FULL, Loading Done

### Section 2 (netmiko) - Script 4 - configuring multiple switches using a 2 files with config

The following script:

- Connects to multiple devices via SSH

- The IP of the devices is manually specified in the script

- It reads a file which contains configuration and applies it to some switches

- It reads another file and applies the configuration to all switches

from netmiko import ConnectHandler

# 5 variables for 5 remote devices

# One Python Dictionary per remote device

ios_l2_sw2 = {

'device_type': 'cisco_ios',

'ip': '192.168.122.82',

'username': 'cisco',

'password': 'cisco'

}

ios_l2_sw3 = {

'device_type': 'cisco_ios',

'ip': '192.168.122.83',

'username': 'cisco',

'password': 'cisco'

}

ios_l2_sw4 = {

'device_type': 'cisco_ios',

'ip': '192.168.122.84',

'username': 'cisco',

'password': 'cisco'

}

ios_l2_sw5 = {

'device_type': 'cisco_ios',

'ip': '192.168.122.85',

'username': 'cisco',

'password': 'cisco'

}

ios_l2_sw6 = {

'device_type': 'cisco_ios',

'ip': '192.168.122.86',

'username': 'cisco',

'password': 'cisco'

}

# Read a file and split its lines

# The 'print' command is optional, but provides visibility

with open('access_switch_configs.txt') as f:

lines = f.read().splitlines()

print (lines)

# A list of the 3 access devices that we want to connect

access_devices = [ios_l2_sw4, ios_l2_sw5, ios_l2_sw6]

# A 'for' loop which connects to the remote devices

# I sents the configuration read from the file

# The 'print' will show us what is applied

for devices in access_devices:

net_connect = ConnectHandler(**devices)

output = net_connect.send_config_set(lines)

print (output)

# Read a file and split its lines

# The 'print' command is optional, but provides visibility

with open('all_switch_configs.txt') as f:

lines = f.read().splitlines()

print (lines)

# A list of all the devices that we want to connect

all_devices = [ios_l2_sw2, ios_l2_sw3, ios_l2_sw4, ios_l2_sw5, ios_l2_sw6]

# A 'for' loop which connects to the remote devices

# I sents the configuration read from the file

# The 'print' will show us what is applied

for devices in all_devices:

net_connect = ConnectHandler(**devices)

output = net_connect.send_config_set(lines)

print (output)

Running the script

root@NetworkAutomation-1:~# python3 netmiko4.py

['vlan 55', ' name VLAN_55', '!', 'router ospf 1', ' network 0.0.0.0 255.255.255.255 area 0 ', '!']

config term

Enter configuration commands, one per line.  End with CNTL/Z.

SW4(config)#vlan 55

SW4(config-vlan)# name VLAN_55

SW4(config-vlan)#!

SW4(config-vlan)#router ospf 1

SW4(config-router)# network 0.0.0.0 255.255.255.255 area 0

SW4(config-router)#!

SW4(config-router)#end

SW4#

config term

Enter configuration commands, one per line.  End with CNTL/Z.

SW5(config)#vlan 55

SW5(config-vlan)# name VLAN_55

SW5(config-vlan)#!

SW5(config-vlan)#router ospf 1

SW5(config-router)# network 0.0.0.0 255.255.255.255 area 0

SW5(config-router)#!

SW5(config-router)#end

SW5#

config term

Enter configuration commands, one per line.  End with CNTL/Z.

SW6(config)#vlan 55

SW6(config-vlan)# name VLAN_55

SW6(config-vlan)#!

SW6(config-vlan)#router ospf 1

SW6(config-router)# network 0.0.0.0 255.255.255.255 area 0

SW6(config-router)#!

SW6(config-router)#end

SW6#

['vlan 66', ' name VLAN_66', '!', 'router eigrp 1', ' network 0.0.0.0 255.255.255.255', '!']

config term

Enter configuration commands, one per line.  End with CNTL/Z.

SW2(config)#vlan 66

SW2(config-vlan)# name VLAN_66

SW2(config-vlan)#!

SW2(config-vlan)#router eigrp 1

SW2(config-router)# network 0.0.0.0 255.255.255.255

SW2(config-router)#!

SW2(config-router)#end

SW2#

config term

Enter configuration commands, one per line.  End with CNTL/Z.

SW3(config)#vlan 66

SW3(config-vlan)# name VLAN_66

SW3(config-vlan)#!

SW3(config-vlan)#router eigrp 1

SW3(config-router)# network 0.0.0.0 255.255.255.255

SW3(config-router)#!

SW3(config-router)#end

SW3#

config term

Enter configuration commands, one per line.  End with CNTL/Z.

SW4(config)#vlan 66

SW4(config-vlan)# name VLAN_66

SW4(config-vlan)#!

SW4(config-vlan)#router eigrp 1

SW4(config-router)# network 0.0.0.0 255.255.255.255

SW4(config-router)#!

SW4(config-router)#end

SW4#

config term

Enter configuration commands, one per line.  End with CNTL/Z.

SW5(config)#vlan 66

SW5(config-vlan)# name VLAN_66

SW5(config-vlan)#!

SW5(config-vlan)#router eigrp 1

SW5(config-router)# network 0.0.0.0 255.255.255.255

SW5(config-router)#!

SW5(config-router)#end

SW5#

config term

Enter configuration commands, one per line.  End with CNTL/Z.

SW6(config)#vlan 66

SW6(config-vlan)# name VLAN_66

SW6(config-vlan)#!

SW6(config-vlan)#router eigrp 1

SW6(config-router)# network 0.0.0.0 255.255.255.255

SW6(config-router)#!

SW6(config-router)#end

SW6#

root@NetworkAutomation-1:~#

Verification

SW2#

*Sep 10 19:24:52.859: %SYS-5-CONFIG_I: Configured from console by cisco on vty0 (192.168.122.220)

SW2#

*Sep 10 19:25:04.520: %DUAL-5-NBRCHANGE: EIGRP-IPv4 1: Neighbor 192.168.122.83 (Vlan1) is up: new adjacency

SW2#

*Sep 10 19:25:20.502: %DUAL-5-NBRCHANGE: EIGRP-IPv4 1: Neighbor 192.168.122.84 (Vlan1) is up: new adjacency

SW2#

*Sep 10 19:25:36.795: %DUAL-5-NBRCHANGE: EIGRP-IPv4 1: Neighbor 192.168.122.85 (Vlan1) is up: new adjacency

SW2#

*Sep 10 19:25:52.974: %DUAL-5-NBRCHANGE: EIGRP-IPv4 1: Neighbor 192.168.122.86 (Vlan1) is up: new adjacency

SW3#show vlan brief

VLAN Name                             Status    Ports

---- -------------------------------- --------- -------------------------------

1    default                          active    Et0/0, Et0/1, Et0/2, Et0/3

Et1/0, Et1/2, Et1/3, Et2/0

Et2/1, Et2/2, Et2/3, Et3/0

Et3/1, Et3/2, Et3/3

2    Python_VLAN 2                    active

3    Python_VLAN 3                    active

66   VLAN_66                          active

SW4#show vlan brief

VLAN Name                             Status    Ports

---- -------------------------------- --------- -------------------------------

1    default                          active    Et0/0, Et0/1, Et0/2, Et0/3

Et1/0, Et1/1, Et1/3, Et2/0

Et2/1, Et2/2, Et2/3, Et3/0

Et3/1, Et3/2, Et3/3

2    Python_VLAN_2                    active

3    Python_VLAN_3                    active

4    Python_VLAN_4                    active

5    Python_VLAN_5                    active

6    Python_VLAN_6                    active

7    Python_VLAN_7                    active

8    Python_VLAN_8                    active

9    Python_VLAN_9                    active

10   Python_VLAN_10                   active

55   VLAN_55                          active

66   VLAN_66                          active

## Network Automation with NAPALM

From https://napalm-automation.net/

What is NAPALM (Network Automation and Programmability Abstraction)?

Napalm is a vendor neutral, cross-platform open source project that provides a unified API to network devices.

Napalm is written in Python and already works with the most popular automation frameworks.

IOS Prerequisites

IOS has no native API to play with, that’s the reason why we used the Netmiko library to interact with it. Having Netmiko installed in your working box is a prerequisite.

Supported Devices  (Sep 2018)

A sample from the past:

Topology

Initial config

R1/SW1:

ip domain-nam mikis.lab

username cisco password cisco

username cisco privilege 15

username mikis password cisco

username mikis privilege 15

crypto key generate rsa

1024

! % Generating 1024 bit RSA keys, keys will be non-exportable...

line vty 0 4

transport input ssh

login local

!

! for the switch will be SVI 1

int e0/0

ip add 192.168.122.72 255.255.255.0

no shut

On NetworkAautomation-1 station enable DHCP Client on eth0 by un-commenting the following:

root@NetworkAutomation-1:/etc/network# nano interfaces

# DHCP config for eth0

auto eth0

iface eth0 inet dhcp

Note about nano: To save press CTRL-X, then 'Y' then Enter

root@NetworkAutomation-1:/etc/network#

NetworkAutomation-1 console is now available... Press RETURN to get started.

udhcpc (v1.24.2) started

Sending discover...

Sending discover...

Sending select for 192.168.122.120...

Lease of 192.168.122.120 obtained, lease time 3600

Verification

root@NetworkAutomation-1:~# ssh -l mikis 192.168.122.72

The authenticity of host '192.168.122.72 (192.168.122.72)' can't be established.

RSA key fingerprint is SHA256:qYzhY6wCSH4TF3/G2gB7+8EPLyyAgcBwyXBCQEOQSCI.

Are you sure you want to continue connecting (yes/no)? yes

Warning: Permanently added '192.168.122.72' (RSA) to the list of known hosts.

Password:

R1#

Install NAPALM

IMPORTANT: Please also make sure you use the following commands to install Python3 Netmiko and NAPALM on the Network Automation Container:

apt-get update

apt-get install python3-pip

apt-get install python3-dev

pip3 install -U netmiko

At this point test netmiko functionality using the following script which runs a 'show' command on a remote device:

root@NetworkAutomation-1:~# cat > netmiko-test.py

# From netmiko module import ConnectHandler

from netmiko import ConnectHandler

# Create a variable that contains a python Dictionary with 4 keys

iosv_l2 = {

'device_type': 'cisco_ios',

'ip': '192.168.122.72',

'username': 'cisco',

'password': 'cisco',

}

# The ** means that the argument is a dictionary

# One * would mean that the argument is a list

# The ConnectHandler will SSH to the device

net_connect = ConnectHandler(**iosv_l2)

# To view something from the remote device over SSH

# Netmiko 'send_command' method to send a 'show' command to the remote device and print the output

output = net_connect.send_command('show ip int brief')

print (output)

root@NetworkAutomation-1:~#

root@NetworkAutomation-1:~# python3 netmiko-test.py

Interface                  IP-Address      OK? Method Status                Protocol

Ethernet0/0                192.168.122.72  YES manual up                    up

Ethernet0/1                unassigned      YES NVRAM  administratively down down

Ethernet0/2                unassigned      YES NVRAM  administratively down down

Ethernet0/3                unassigned      YES NVRAM  administratively down down

Ethernet1/0                unassigned      YES NVRAM  administratively down down

Ethernet1/1                unassigned      YES NVRAM  administratively down down

Ethernet1/2                unassigned      YES NVRAM  administratively down down

Ethernet1/3                unassigned      YES NVRAM  administratively down down

Serial2/0                  unassigned      YES NVRAM  administratively down down

Serial2/1                  unassigned      YES NVRAM  administratively down down

Serial2/2                  unassigned      YES NVRAM  administratively down down

Serial2/3                  unassigned      YES NVRAM  administratively down down

Serial3/0                  unassigned      YES NVRAM  administratively down down

Serial3/1                  unassigned      YES NVRAM  administratively down down

Serial3/2                  unassigned      YES NVRAM  administratively down down

Serial3/3                  unassigned      YES NVRAM  administratively down down

Loopback0                  1.1.1.1         YES manual up                    up

root@NetworkAutomation-1:~#

pip3 install -U napalm

pip3 install -U simplejson

### Section 3 (napalm) - Script 1 - Get remote device info using NAPALM

Topology

The following script:

- Connects to a remote IOS device using SSH (Netmiko)

- Gets generic info from the remote device

- It prints the output as a Python Dictionary

root@NetworkAutomation-1:~# cat > netmiko-test.py

# From napalm module import get_network_driver function

from napalm import get_network_driver

# Run get_network_driver and speficy the remote device driver

driver = get_network_driver('ios')

# Connect the the remote device specifying 'IP', 'User', 'Pass':

device = driver('192.168.122.72', 'mikis', 'cisco')

device.open()

# Collect information from the remote device and print it

ios_output = device.get_facts()

print (ios_output)

Running the script

root@NetworkAutomation-1:~# python3 napalm1.py

{'serial_number': '2048002', 'os_version': 'Linux Software (I86BI_LINUX-ADVENTERPRISEK9-M), Version 15.4(3)M9, DEVELOPMENT TEST SOFTWARE', 'fqdn': 'R1.mikis.lab', 'model': 'Unknown', 'interface_list': ['Ethernet0/0', 'Ethernet0/1', 'Ethernet0/2', 'Ethernet0/3', 'Ethernet1/0', 'Ethernet1/1', 'Ethernet1/2', 'Ethernet1/3', 'Serial2/0', 'Serial2/1', 'Serial2/2', 'Serial2/3', 'Serial3/0', 'Serial3/1', 'Serial3/2', 'Serial3/3', 'Loopback0'], 'hostname': 'R1', 'uptime': 2340, 'vendor': 'Cisco'}

root@NetworkAutomation-1:~#

### Section 3 (napalm) - Script 2 - Get remote device info using NAPALM (improved output)

Topology

The following script:

- Connects to a remote IOS device using SSH (Netmiko)

- Gets generic info from the remote device

- It gets the remote output as a Python Dictionary, but it prints it line-by-line

root@NetworkAutomation-1:~# cat > napalm2.py

# From napalm module import get_network_driver function

from napalm import get_network_driver

# Run get_network_driver and speficy the remote device driver

driver = get_network_driver('ios')

# Connect the the remote device specifying 'IP', 'User', 'Pass':

device = driver('192.168.122.72', 'mikis', 'cisco')

device.open()

# Collect information from the remote device and print it

ios_output = device.get_facts()

# Print the Dictionary line-by-line

for keys, values in ios_output.items():

print (keys, ':', values)

Running the script

root@NetworkAutomation-1:~# python3 napalm2.py

uptime : 9480

fqdn : R1.mikis.lab

vendor : Cisco

model : Unknown

serial_number : 2048002

hostname : R1

interface_list : ['Ethernet0/0', 'Ethernet0/1', 'Ethernet0/2', 'Ethernet0/3', 'Ethernet1/0', 'Ethernet1/1', 'Ethernet1/2', 'Ethernet1/3', 'Serial2/0', 'Serial2/1', 'Serial2/2', 'Serial2/3', 'Serial3/0', 'Serial3/1', 'Serial3/2', 'Serial3/3', 'Loopback0']

os_version : Linux Software (I86BI_LINUX-ADVENTERPRISEK9-M), Version 15.4(3)M9, DEVELOPMENT TEST SOFTWARE

root@NetworkAutomation-1:~#

### Section 3 (napalm) - Script 3 - Get remote device info using NAPALM (improved output using JSON)

Topology

The following script:

- Connects to a remote IOS device using SSH (Netmiko)

- Gets generic info from the remote device

- It gets the remote output as a Python Dictionary and it prints it using JSON formatting

root@NetworkAutomation-1:~# cat > napalm1-json.py

# From napalm module import get_network_driver function

from napalm import get_network_driver

# Import JSON for proper formatting

import json

# Run get_network_driver and speficy the remote device driver

driver = get_network_driver('ios')

# Connect the the remote device specifying 'IP', 'User', 'Pass':

device = driver('192.168.122.72', 'mikis', 'cisco')

device.open()

# Collect information from the remote device and print it

ios_output = device.get_facts()

# Print the Dictionary in a JSON format

print (json.dumps(ios_output, indent=4))

Running the script

root@NetworkAutomation-1:~# python3 napalm1-json.py

{

"vendor": "Cisco",

"interface_list": [

"Ethernet0/0",

"Ethernet0/1",

"Ethernet0/2",

"Ethernet0/3",

"Ethernet1/0",

"Ethernet1/1",

"Ethernet1/2",

"Ethernet1/3",

"Serial2/0",

"Serial2/1",

"Serial2/2",

"Serial2/3",

"Serial3/0",

"Serial3/1",

"Serial3/2",

"Serial3/3",

"Loopback0"

],

"fqdn": "R1.mikis.lab",

"os_version": "Linux Software (I86BI_LINUX-ADVENTERPRISEK9-M), Version 15.4(3)M9, DEVELOPMENT TEST SOFTWARE",

"serial_number": "2048002",

"hostname": "R1",

"model": "Unknown",

"uptime": 10080

}

root@NetworkAutomation-1:~#

### Section 3 (napalm) - Script 4 - Get more remote device info using NAPALM (improved output using JSON)

Topology

The following script:

- Connects to a remote IOS device using SSH (Netmiko)

- Gets generic info from the remote device

- It gets the remote output as a Python Dictionary and it prints it using JSON formatting

root@NetworkAutomation-1:~# cat > napalm1-json2.py

# From napalm module import get_network_driver function

from napalm import get_network_driver

# Import JSON for proper formatting

import json

# Run get_network_driver and speficy the remote device driver

driver = get_network_driver('ios')

# Connect the the remote device specifying 'IP', 'User', 'Pass':

device = driver('192.168.122.72', 'mikis', 'cisco')

device.open()

# Collect information from the remote device and print it

ios_output = device.get_facts()

# Print the Dictionary in a JSON format

print (json.dumps(ios_output, indent=4))

# Collect information from the remote device and print it

ios_output = device.get_arp_table()

# Print the Dictionary in a JSON format

print (json.dumps(ios_output, indent=4))

# Collect information from the remote device and print it

ios_output = device.get_config()

# Print the Dictionary in a JSON format

print (json.dumps(ios_output, indent=4))

# Collect information from the remote device and print it

ios_output = device.get_interfaces_counters()

# Print the Dictionary in a JSON format

print (json.dumps(ios_output, indent=4))

Running the script

root@NetworkAutomation-1:~# python3 napalm1-json2.py

{

"os_version": "Linux Software (I86BI_LINUX-ADVENTERPRISEK9-M), Version 15.4(3)M9, DEVELOPMENT TEST SOFTWARE",

"serial_number": "2048002",

"vendor": "Cisco",

"uptime": 10380,

"fqdn": "R1.mikis.lab",

"interface_list": [

"Ethernet0/0",

"Ethernet0/1",

"Ethernet0/2",

"Ethernet0/3",

"Ethernet1/0",

"Ethernet1/1",

"Ethernet1/2",

"Ethernet1/3",

"Serial2/0",

"Serial2/1",

"Serial2/2",

"Serial2/3",

"Serial3/0",

"Serial3/1",

"Serial3/2",

"Serial3/3",

"Loopback0"

],

"model": "Unknown",

"hostname": "R1"

}

[

{

"age": 0.0,

"interface": "Ethernet0/0",

"ip": "192.168.122.72",

"mac": "AA:BB:CC:00:02:00"

},

{

"age": 158.0,

"interface": "Ethernet0/0",

"ip": "192.168.122.120",

"mac": "92:39:C3:42:6A:4C"

}

]

{

"startup": "Using 1985 out of 32768 bytes\n!\n! Last configuration change at 08:45:37 UTC Tue Sep 11 2018\n!\nversion 15.4\nservice timestamps debug datetime msec\nservice timestamps log datetime msec\nno service password-encryption\n!\nhostname R1\n!\nboot-start-marker\nboot-end-marker\n!\n!\n!\nno aaa new-model\n!\n!\n!\nmmi polling-interval 60\nno mmi auto-configure\nno mmi pvc\nmmi snmp-timeout 180\n!\n!\n!\n!\n!\nno ip icmp rate-limit unreachable\n!\n!\n!\n!\n!\n!\n!\n!\n\n\n!\n!\n!\n!\nno ip domain lookup\nip domain name mikis.lab\nip cef\nno ipv6 cef\n!\nmultilink bundle-name authenticated\n!\n!\n!\n!\n!\n!\n!\ncts logging verbose\n!\n!\nusername cisco privilege 15 password 0 cisco\nusername mikis privilege 15 password 0 cisco\n!\nredundancy\n!\n!\nip tcp synwait-time 5\n! \n!\n!\n!\n!\n!\n!\n!\n!\n!\n!\n!\ninterface Ethernet0/0\n ip address 192.168.122.72 255.255.255.0\n!\ninterface Ethernet0/1\n no ip address\n shutdown\n!\ninterface Ethernet0/2\n no ip address\n shutdown\n!\ninterface Ethernet0/3\n no ip address\n shutdown\n!\ninterface Ethernet1/0\n no ip address\n shutdown\n!\ninterface Ethernet1/1\n no ip address\n shutdown\n!\ninterface Ethernet1/2\n no ip address\n shutdown\n!\ninterface Ethernet1/3\n no ip address\n shutdown\n!\ninterface Serial2/0\n no ip address\n shutdown\n serial restart-delay 0\n!\ninterface Serial2/1\n no ip address\n shutdown\n serial restart-delay 0\n!\ninterface Serial2/2\n no ip address\n shutdown\n serial restart-delay 0\n!\ninterface Serial2/3\n no ip address\n shutdown\n serial restart-delay 0\n!\ninterface Serial3/0\n no ip address\n shutdown\n serial restart-delay 0\n!\ninterface Serial3/1\n no ip address\n shutdown\n serial restart-delay 0\n!\ninterface Serial3/2\n no ip address\n shutdown\n serial restart-delay 0\n!\ninterface Serial3/3\n no ip address\n shutdown\n serial restart-delay 0\n!\nip forward-protocol nd\n!\n!\nno ip http server\nno ip http secure-server\n!\n!\n!\n!\ncontrol-plane\n!\n!\n!\n!\n!\n!\n!\n!\nline con 0\n exec-timeout 0 0\n privilege level 15\n logging synchronous\nline aux 0\n exec-timeout 0 0\n privilege level 15\n logging synchronous\nline vty 0 4\n login local\n transport input ssh\n!\n!\nend",

"running": "Building configuration...\n\nCurrent configuration : 2052 bytes\n!\n! Last configuration change at 08:54:25 UTC Tue Sep 11 2018 by cisco\n!\nversion 15.4\nservice timestamps debug datetime msec\nservice timestamps log datetime msec\nno service password-encryption\n!\nhostname R1\n!\nboot-start-marker\nboot-end-marker\n!\n!\n!\nno aaa new-model\n!\n!\n!\nmmi polling-interval 60\nno mmi auto-configure\nno mmi pvc\nmmi snmp-timeout 180\n!\n!\n!\n!\n!\nno ip icmp rate-limit unreachable\n!\n!\n!\n!\n!\n!\n!\n!\n\n\n!\n!\n!\n!\nno ip domain lookup\nip domain name mikis.lab\nip cef\nno ipv6 cef\n!\nmultilink bundle-name authenticated\n!\n!\n!\n!\n!\n!\n!\ncts logging verbose\n!\n!\nusername cisco privilege 15 password 0 cisco\nusername mikis privilege 15 password 0 cisco\n!\nredundancy\n!\n!\nip tcp synwait-time 5\n! \n!\n!\n!\n!\n!\n!\n!\n!\n!\n!\n!\ninterface Loopback0\n ip address 1.1.1.1 255.255.255.255\n!\ninterface Ethernet0/0\n ip address 192.168.122.72 255.255.255.0\n!\ninterface Ethernet0/1\n no ip address\n shutdown\n!\ninterface Ethernet0/2\n no ip address\n shutdown\n!\ninterface Ethernet0/3\n no ip address\n shutdown\n!\ninterface Ethernet1/0\n no ip address\n shutdown\n!\ninterface Ethernet1/1\n no ip address\n shutdown\n!\ninterface Ethernet1/2\n no ip address\n shutdown\n!\ninterface Ethernet1/3\n no ip address\n shutdown\n!\ninterface Serial2/0\n no ip address\n shutdown\n serial restart-delay 0\n!\ninterface Serial2/1\n no ip address\n shutdown\n serial restart-delay 0\n!\ninterface Serial2/2\n no ip address\n shutdown\n serial restart-delay 0\n!\ninterface Serial2/3\n no ip address\n shutdown\n serial restart-delay 0\n!\ninterface Serial3/0\n no ip address\n shutdown\n serial restart-delay 0\n!\ninterface Serial3/1\n no ip address\n shutdown\n serial restart-delay 0\n!\ninterface Serial3/2\n no ip address\n shutdown\n serial restart-delay 0\n!\ninterface Serial3/3\n no ip address\n shutdown\n serial restart-delay 0\n!\nip forward-protocol nd\n!\n!\nno ip http server\nno ip http secure-server\n!\n!\n!\n!\ncontrol-plane\n!\n!\n!\n!\n!\n!\n!\n!\nline con 0\n exec-timeout 0 0\n privilege level 15\n logging synchronous\nline aux 0\n exec-timeout 0 0\n privilege level 15\n logging synchronous\nline vty 0 4\n login local\n transport input ssh\n!\n!\nend",

"candidate": ""

}

{

"Serial2/1": {

"tx_unicast_packets": 0,

"tx_errors": 0,

"rx_broadcast_packets": 0,

"rx_multicast_packets": 0,

"rx_discards": 0,

"tx_discards": 0,

"rx_errors": 0,

"rx_unicast_packets": 0,

"tx_multicast_packets": -1,

"tx_octets": 0,

"tx_broadcast_packets": -1,

"rx_octets": 0

},

...

"Serial3/3": {

"tx_unicast_packets": 0,

"tx_errors": 0,

"rx_broadcast_packets": 0,

"rx_multicast_packets": 0,

"rx_discards": 0,

"tx_discards": 0,

"rx_errors": 0,

"rx_unicast_packets": 0,

"tx_multicast_packets": -1,

"tx_octets": 0,

"tx_broadcast_packets": -1,

"rx_octets": 0

}

}

root@NetworkAutomation-1:~#

### Section 3 (napalm) - Script 5 - Get BGP neighbor info using NAPALM  and print the output using JSON

Topology

SW1#show ip bgp summary | b Nei

Neighbor        V           AS MsgRcvd MsgSent   TblVer  InQ OutQ Up/Down  State/PfxRcd

192.168.122.72  4        65000      18      18        3    0    0 00:12:08        1

R1#show ip bgp summary  | b Nei

Neighbor        V           AS MsgRcvd MsgSent   TblVer  InQ OutQ Up/Down  State/PfxRcd

192.168.122.82  4        65000      17      17        5    0    0 00:11:43        1

The following script:

- Connects to multiple remote IOS device using SSH (Netmiko)

- Gets BGP info from the remote devices

- It gets the remote outputs as a Python Dictionaries and prints it using JSON formatting

root@NetworkAutomation-1:~# cat > napalm3.py

# Import a module and a fuction from module

import json

from napalm import get_network_driver

# A list with the IPs of the remote devices

bgplist = ['192.168.122.72', '192.168.122.82']

# A 'for' loop which connects to remote devices

for ip in bgplist:

print ("Connecting to " + str(ip))

# The remote device type

driver = get_network_driver('ios')

# Conenct to remote device

device = driver(ip, 'mikis', 'cisco')

device.open()

# Get BGP neighbor info and print it in JSON format

bgp_neighbors = device.get_bgp_neighbors()

print (json.dumps(bgp_neighbors, indent=4))

device.close()

Running the script

root@NetworkAutomation-1:~# python3 napalm3.py

Connecting to 192.168.122.72

{

"global": {

"router_id": "2.2.2.2",

"peers": {

"192.168.122.82": {

"description": "",

"is_up": true,

"local_as": 65000,

"is_enabled": true,

"remote_id": "1.1.1.1",

"address_family": {

"ipv4": {

"received_prefixes": 1,

"sent_prefixes": 1,

"accepted_prefixes": 1

}

},

"remote_as": 65000,

"uptime": 15

}

}

}

}

Connecting to 192.168.122.82

{

"global": {

"router_id": "1.1.1.1",

"peers": {

"192.168.122.72": {

"description": "",

"is_up": true,

"local_as": 65000,

"is_enabled": true,

"remote_id": "2.2.2.2",

"address_family": {

"ipv4": {

"received_prefixes": 1,

"sent_prefixes": 1,

"accepted_prefixes": 1

}

},

"remote_as": 65000,

"uptime": 24

}

}

}

}

root@NetworkAutomation-1:~#

### Section 3 (napalm) - Script 6 - Making config changes to remote devices using NAPALM

Topology

The following script:

- Connects to a device using SSH (Netmiko) and sends configuration using Napalm

The config we want to apply (merge):

root@NetworkAutomation-1:~# cat R1.cfg

router rip

no auto-summary

version 2

network 192.168.122.0

!

ip name-server 8.8.8.8

!

root@NetworkAutomation-1:~#

Note

I tried to run a config script to an IOS device and got:

root@NetworkAutomation-1:~# python3 napalm_config1.py

Accessing 192.168.122.72

Traceback (most recent call last):

File "napalm_config1.py", line 12, in <module>

device.load_merge_candidate(filename='R1.cfg')

File "/usr/local/lib/python3.5/dist-packages/napalm/ios/ios.py", line 268, in load_merge_candidate

file_system=self.dest_file_system)

File "/usr/local/lib/python3.5/dist-packages/napalm/ios/ios.py", line 238, in _load_candidate_wrapper

file_system=file_system)

File "/usr/local/lib/python3.5/dist-packages/napalm/ios/ios.py", line 513, in _scp_file

file_system=file_system, TransferClass=FileTransfer)

File "/usr/local/lib/python3.5/dist-packages/napalm/ios/ios.py", line 548, in _xfer_file

if not transfer.verify_space_available():

File "/usr/local/lib/python3.5/dist-packages/netmiko/scp_handler.py", line 146, in verify_space_available

space_avail = self.remote_space_available(search_pattern=search_pattern)

File "/usr/local/lib/python3.5/dist-packages/netmiko/scp_handler.py", line 105, in remote_space_available

return int(match.group(1))

AttributeError: 'NoneType' object has no attribute 'group'

From:

https://github.com/napalm-automation/napalm/issues/612

napalm-ios requires a file system to transfer a file into for any of the configuration operations.

and

R2(config)#ip scp server enable

root@NetworkAutomation-1:~# cat > napalm_config1.py

import json

from napalm import get_network_driver

# Connect to the remote device

driver = get_network_driver('ios')

device = driver('192.168.122.73', 'mikis', 'cisco')

device.open()

# Push configuration from a file to the remote device

print ("Accessing 192.168.122.73")

device.load_merge_candidate(filename='R1.cfg')

# If you are happy with the changes you can commit them:

device.commit_config()

device.close()

Running the script

root@NetworkAutomation-1:~# python3 napalm_config1.py

Accessing 192.168.122.73

root@NetworkAutomation-1:~#

Verification

R2#sh run | s router

router rip

version 2

network 192.168.122.0

no auto-summary

R2#

R2#

R2#sh run | s name

hostname R2

ip domain name mikis.lab

ip name-server 8.8.8.8

multilink bundle-name authenticated

username mikis privilege 15 password 0 cisco

### Section 3 (napalm) - Script 6 - Check for diff and make config changes to remote devices using NAPALM

Topology

The following script:

- Connects to a device using SSH (Netmiko) and sends configuration using Napalm

- If the provided config is different than the existing it commits it

- If the provided config is the same as the existing it discards the changes

root@NetworkAutomation-1:~# cat > napalm_config2.py

import json

from napalm import get_network_driver

# Open an SSH session to the remote device

driver = get_network_driver("ios")

device = driver("192.168.122.73", "mikis", "cisco")

device.open()

# Read the candidate configuration from a file

print ("Accessing 192.168.122.73")

device.load_merge_candidate(filename="R1.cfg")

# Check for differences

# Commit if there are differences

# Otherwise discard

diffs = device.compare_config()

if len(diffs) > 0:

print(diffs)

device.commit_config()

else:

print("No changes required.")

device.discard_config()

device.close()

root@NetworkAutomation-1:~#

Running the script

If there are differences the script shows:

root@NetworkAutomation-1:~# python3 napalm_config2.py

Accessing 192.168.122.73

+router rip

- no auto

root@NetworkAutomation-1:~#

If there are no differences the script shows:

root@NetworkAutomation-1:~# python3 napalm_config2.py

Accessing 192.168.122.73

No changes required.

root@NetworkAutomation-1:~#

### Section 3 (napalm) - Script 7 - Check for config diff multiple devices using NAPALM

Topology

The following script:

- Connects to remote devices using SSH (Netmiko)

- Compares the remote device config with their file configs

- If the file config has config that does not exist in the remote device it prints the differences

root@NetworkAutomation-1:~# cat > napalm_check_diff.py

from napalm import get_network_driver

# The devices and their file configs are mentioned in a Python Dictionary

devices_dictionary = {"192.168.122.73": "R2.cfg", "192.168.122.82": "SW1.cfg"}

devices = devices_dictionary.items()

# A 'for' loop which runs through the dictionary items

# It gets the IP of the device and connects via SSH

# Then it compares its configuration with the file configuration

# If there is a difference it prints it

# Otherwise it does nothing

for ip, config in devices:

# Open an SSH session to the remote device

driver = get_network_driver("ios")

device = driver(str(ip), "mikis", "cisco")

device.open()

# Read the candidate configuration from a file

print ("Accessing %r and reading %r" % (ip,config))

device.load_merge_candidate(filename=str(config))

# Check for differences

diffs = device.compare_config()

if len(diffs) > 0:

print(diffs)

else:

print("No changes required.")

device.close()

Running the script

root@NetworkAutomation-1:~# python3 napalm_check_diff.py

Accessing '192.168.122.73' and reading 'R2.cfg'

+interface Loopback2

+ ip address 2.2.2.2 255.255.255.255

Accessing '192.168.122.82' and reading 'SW1.cfg'

No changes required.

root@NetworkAutomation-1:~#

## Section 8 - Iterations with Netmiko

### Section 8 (iterations with Netmiko) - Iteration 1 - Send a 'show' command

Topology

The following script:

- Connects to a single remote device using SSH (Netmiko)

- Runs a 'show' command and prints the output

root@NetworkAutomation-1:~# cat > netmiko_iter1.py

# Import a function from netmiko module

from netmiko import ConnectHandler

# A dictionary for the remote device

device_sw1 = {

'device_type': 'cisco_ios',

'ip': '192.168.122.11',

'username': 'mikis',

'password': 'cisco'

}

# Connect to the remote device using SSH

# Send a command and print the output

net_connect = ConnectHandler(**device_sw1)

output = net_connect.send_command('show ip int brief')

print(output)

Running the script

root@NetworkAutomation-1:~# python3 netmiko_iter1.py

Interface              IP-Address      OK? Method Status                Protocol

Ethernet0/0            unassigned      YES unset  up                    up

Ethernet0/1            unassigned      YES unset  up                    up

Ethernet0/2            unassigned      YES unset  up                    up

Ethernet0/3            unassigned      YES unset  up                    up

Ethernet1/0            unassigned      YES unset  up                    up

Ethernet1/1            unassigned      YES unset  up                    up

Ethernet1/2            unassigned      YES unset  up                    up

Ethernet1/3            unassigned      YES unset  up                    up

Ethernet2/0            unassigned      YES unset  up                    up

Ethernet2/1            unassigned      YES unset  up                    up

Ethernet2/2            unassigned      YES unset  up                    up

Ethernet2/3            unassigned      YES unset  up                    up

Ethernet3/0            unassigned      YES unset  up                    up

Ethernet3/1            unassigned      YES unset  up                    up

Ethernet3/2            unassigned      YES unset  up                    up

Ethernet3/3            unassigned      YES unset  up                    up

Vlan1                  192.168.122.11  YES NVRAM  up                    up

root@NetworkAutomation-1:~#

### Section 8 (iterations with Netmiko) - Iteration 2 - Send commands from a file

Topology

The following script:

- Connects to a single remote device using SSH (Netmiko)

- Reads commands from a file and sends them to the remote device

root@NetworkAutomation-1:~# cat > netmiko_iter2.py

# Import a function from netmiko module

from netmiko import ConnectHandler

# Open a file, read it and split its lines

with open('commands_file') as f:

commands_to_send = f.read().splitlines()

# Dictionary for device SW2

device_sw2 = {

'device_type': 'cisco_ios',

'ip': '192.168.122.12',

'username': 'mikis',

'password': 'cisco'

}

devices = [device_sw2]

for device in devices:

net_connect = ConnectHandler(**device)

output = net_connect.send_config_set(commands_to_send)

print (output)

Running the script

root@NetworkAutomation-1:~# python3 netmiko_iter2.py

config term

Enter configuration commands, one per line.  End with CNTL/Z.

SW2(config)#router eigrp 1

SW2(config-router)# network 0.0.0.0 255.255.255.255

SW2(config-router)#end

SW2#

root@NetworkAutomation-1:~#

### Section 8 (iterations with Netmiko) - Iteration 3 - Send commands from a file to multiple devices

Topology

The following script:

- Reads the IPs of devices from a file and connects using SSH (Netmiko)

- Reads commands from a file and sends them to the remote devices

root@NetworkAutomation-1:~# cat > netmiko_iter3.py

# import function ConnectHandler from netmiko module

from netmiko import ConnectHandler

# Open a file which contains commands and read it spliting the lines

with open('commands_file') as f:

commands_list = f.read().splitlines()

# Open a file which contains the remote device IPs

with open('devices_file') as f:

devices_list = f.read().splitlines()

# Run a loop through the device IPs

for devices in devices_list:

print ('Connecting to device: ' + devices)

ip_address_of_device = devices

device = {

'device_type': 'cisco_ios',

'ip': ip_address_of_device,

'username': 'mikis',

'password': 'cisco'

}

# Connect to the remote devices using Netmiko

# Send the commands and print the output

net_connect = ConnectHandler(**device)

output = net_connect.send_config_set(commands_list)

print (output)

Running the script

root@NetworkAutomation-1:~# python3 netmiko_iter3.py

Connecting to device: 192.168.122.21

config term

Enter configuration commands, one per line.  End with CNTL/Z.

R1(config)#router ospf 1

R1(config-router)# network 0.0.0.0 255.255.255.255 area 0

R1(config-router)#end

R1#

Connecting to device: 192.168.122.22

config term

Enter configuration commands, one per line.  End with CNTL/Z.

R2(config)#router ospf 1

R2(config-router)# network 0.0.0.0 255.255.255.255 area 0

R2(config-router)#end

R2#

root@NetworkAutomation-1:~#

### Section 8 (iterations with Netmiko) - Iteration 4 - Send commands from a file to multiple devices with user prompt

Topology

The following script:

- Reads the IPs of devices from a file and connects using SSH (Netmiko)

- Reads commands from a file and sends them to the remote devices

- The user is prompted for SSH username and password

root@NetworkAutomation-1:~# cat > netmiko_iter4.py

from getpass import getpass

from netmiko import ConnectHandler

# Prompt for username and password

provided_username = input('Enter your SSH username: ')

provided_password = getpass()

# Open and read commands from a file

with open('commands_file') as f:

commands_list = f.read().splitlines()

# Open and read IPs from a file

with open('devices_file') as f:

devices_list = f.read().splitlines()

# Create a dictionary entry for each remote device

# Using the provided credentials and IP from the file

for devices in devices_list:

print ('Connecting to device: ' + devices)

ip_address_of_device = devices

device = {

'device_type': 'cisco_ios',

'ip': ip_address_of_device,

'username': provided_username,

'password': provided_password

}

# Connect to the remote devices using the provides credentials

net_connect = ConnectHandler(**device)

output = net_connect.send_config_set(commands_list)

print(output)

Running the script

root@NetworkAutomation-1:~# python3 netmiko_iter4.py

Enter your SSH username: mikis

Password:

Connecting to device: 192.168.122.21

config term

Enter configuration commands, one per line.  End with CNTL/Z.

R1(config)#router ospf 1

R1(config-router)# network 0.0.0.0 255.255.255.255 area 0

R1(config-router)#end

R1#

Connecting to device: 192.168.122.22

config term

Enter configuration commands, one per line.  End with CNTL/Z.

R2(config)#router ospf 1

R2(config-router)# network 0.0.0.0 255.255.255.255 area 0

R2(config-router)#end

R2#

root@NetworkAutomation-1:~#

### Section 8 (iterations with Netmiko) - Iteration 5 - Send commands from a file to multiple devices with user prompt and error handling

Topology

The following script:

- Reads the IPs of devices from a file and connects using SSH (Netmiko)

- Reads commands from a file and sends them to the remote devices

- The user is prompted for SSH username and password and there is error handling

root@NetworkAutomation-1:~# cat > netmiko_iter5.py

from getpass import getpass

from netmiko import ConnectHandler

from netmiko.ssh_exception import NetMikoTimeoutException

from netmiko.ssh_exception import AuthenticationException

from paramiko.ssh_exception import SSHException

# Promt for username and password

provided_username = input("Enter your SSH username: ")

provided_password = getpass()

# Open and read a file which contains IOS commands

with open("commands_file") as f:

commands_list = f.read().splitlines()

# Open and read a file which contains IPs of remote devices

with open("devices_file") as f:

devices_list = f.read().splitlines()

# Loop which creates a dictionary for each remote device

# Then it connects to each remote device using Netmiko

# The remote connection provides Error Handling

for devices in devices_list:

print ("Connecting to device: " + devices)

ip_address_of_device = devices

device = {

'device_type': 'cisco_ios',

'ip': ip_address_of_device,

'username': provided_username,

'password': provided_password

}

# Try to use Netmiko to SSH to remote device along with Error handling

try:

net_connect = ConnectHandler(**device)

# In case authentication fails

except (AuthenticationException):

print ("Authentication failure: " + ip_address_of_device)

continue

# In case remote device does not respond

except (NetMikoTimeoutException):

print ("Timeout to device: " + ip_address_of_device)

continue

# In case the file is empty

except (EOFError):

print ("End of file while attempting device: " + ip_address_of_device)

continue

# In case there is SSH issue

except (SSHException):

print ("SSH Issue. Are you sure SSH is enabled? " + ip_address_of_device)

continue

except Exception as unknown_error:

print ("Some other error: " + unknown_error)

continue

output = net_connect.send_config_set(commands_list)

print(output)

Running the script

Attempt 1 - all good

root@NetworkAutomation-1:~# python3 netmiko_iter5.py

Enter your SSH username: mikis

Password:

Connecting to device: 192.168.122.21

config term

Enter configuration commands, one per line.  End with CNTL/Z.

R1(config)#router ospf 1

R1(config-router)# network 0.0.0.0 255.255.255.255 area 0

R1(config-router)#end

R1#

Connecting to device: 192.168.122.22

config term

Enter configuration commands, one per line.  End with CNTL/Z.

R2(config)#router ospf 1

R2(config-router)# network 0.0.0.0 255.255.255.255 area 0

R2(config-router)#end

R2#

root@NetworkAutomation-1:~#

Attempt 2 - Wrong password

root@NetworkAutomation-1:~# python3 netmiko_iter5.py

Enter your SSH username: mikis

Password:

Connecting to device: 192.168.122.21

Authentication failure: 192.168.122.21

Connecting to device: 192.168.122.22

Authentication failure: 192.168.122.22

Attempt 3 - Wrong IPs in the device file

root@NetworkAutomation-1:~# python3 netmiko_iter5.py

Enter your SSH username: mikis

Password:

Connecting to device: #192.168.122.21

Timeout to device: #192.168.122.21

Connecting to device: #192.168.122.22

Timeout to device: #192.168.122.22

## Section 9 - Scaling Python scripts

### Section 9 (scaling Netmiko) - Encrypt a passwords file using simple-crypt

Topology

The following script:

- Reads a CSV file that contains credentials and ecnrypts it

root@NetworkAutomation-1:~# cat > encrypt_passwords1.py

from simplecrypt import encrypt, decrypt

from pprint import pprint

import csv

import json

# Step 1 - Get user input

# We ask for filename which contains the user passwords and a key for encryption

dc_in_filename  = input("Input CSV filename (device-creds) : ") or "device-creds"

key                             = input("Encryption key (cisco) : ") or "cisco"

# Step 2 - Read the device credentials from a CSV file and put them into a list

with open(dc_in_filename, 'r') as dc_in:

device_creds_reader = csv.reader(dc_in)

device_creds_list = [device for device in device_creds_reader]

# Step 3 - print the list created in step 2

print ("\n----- device_creds ------------------")

pprint(device_creds_list)

# Encrypt the device credentials using the key

encrypted_dc_out_filename = input("Output ecnrypted filename (encrypted-device-creds): ") or "encrypted-device-creds"

# Open the file for read/write as binary

# Encrypt the file in JSON format

with open(encrypted_dc_out_filename, 'wb') as dc_out:

dc_out.write(encrypt(key, json.dumps(device_creds_list)))

print ("woohoo, I have encrypted the file!")

# For verification, open the encrypted file

# Store the decrypted info in 'device_creds_json' variable

print ("\n... Decrypting the encrypted file ...\n")

with open(encrypted_dc_out_filename, 'rb') as device_creds_file:

device_creds_json = decrypt(key, device_creds_file.read())

# Decode and put into a list the variable info

# Then print it

device_creds_list = json.loads(device_creds_json.decode('utf-8'))

pprint(device_creds_list)

# and display it

print ("\n----- confirm: device_creds json in ---------------------")

device_creds = { dev[0]:dev for dev in device_creds_list}

pprint(device_creds)

Running the script

root@NetworkAutomation-1:~# python3 encrypt_passwords1.py

Input CSV filename (device-creds) : pass

Encryption key (cisco) : key2

----- device_creds ------------------

[['192.168.122.82', 'mikis', 'cisco'],

['192.168.122.72', 'mikis', 'cisco'],

['192.168.122.73', 'mikis', 'cisco']]

Output ecnrypted filename (encrypted-device-creds): encrypted3

woohoo, I have encrypted the file!

... Decrypting the encrypted file ...

[['192.168.122.82', 'mikis', 'cisco'],

['192.168.122.72', 'mikis', 'cisco'],

['192.168.122.73', 'mikis', 'cisco']]

----- confirm: device_creds json in ---------------------

{'192.168.122.72': ['192.168.122.72', 'mikis', 'cisco'],

'192.168.122.73': ['192.168.122.73', 'mikis', 'cisco'],

'192.168.122.82': ['192.168.122.82', 'mikis', 'cisco']}

root@NetworkAutomation-1:~#

root@NetworkAutomation-1:~# ls -alt | grep encr

-rw-r--r-- 1 root root   182 Sep 13 12:05 encrypted3

-rw-r--r-- 1 root root  1691 Sep 13 12:05 encrypt_passwords1.py

If I try to see the contents of the encrypted3 file the VM crashes

### Section 9 (scaling Netmiko) - Measuring the script execution time in Netmiko sequential deployment

Topology

The following script:

- Connects to remote devices sequentially using SSH (Netmiko)

- Measures the script total execution time

root@NetworkAutomation-1:~# cat > measure_time_sequential.py

from netmiko import ConnectHandler

from time import time

ios_sw1 = {

'device_type': 'cisco_ios',

'ip': '192.168.122.82',

'username': 'mikis',

'password': 'cisco'

}

ios_r1 = {

'device_type': 'cisco_ios',

'ip': '192.168.122.72',

'username': 'mikis',

'password': 'cisco'

}

ios_r2 = {

'device_type': 'cisco_ios',

'ip': '192.168.122.73',

'username': 'mikis',

'password': 'cisco'

}

# A list with all devices

all_devices = [ios_sw1, ios_r1, ios_r2]

# The main script starts

starting_time = time()

# 'for' loop to connect to remote devices

for device in all_devices:

print("\nConnecting to device " + device["ip"])

net_connect = ConnectHandler(**device)

output = net_connect.send_command("show ip int brief")

print(output)

# Print the total execution time

print ('\n---- End get config sequential, elapsed time=', time()-starting_time)

Running the script

root@NetworkAutomation-1:~# python3 measure_time_sequential.py

Connecting to device 192.168.122.82

Interface              IP-Address      OK? Method Status                Protocol

Ethernet0/0            unassigned      YES unset  up                    up

Ethernet0/1            unassigned      YES unset  up                    up

Ethernet0/2            unassigned      YES unset  up                    up

Ethernet0/3            unassigned      YES unset  up                    up

Ethernet1/0            unassigned      YES unset  up                    up

Ethernet1/1            unassigned      YES unset  up                    up

Ethernet1/2            unassigned      YES unset  up                    up

Ethernet1/3            unassigned      YES unset  up                    up

Ethernet2/0            unassigned      YES unset  up                    up

Ethernet2/1            unassigned      YES unset  up                    up

Ethernet2/2            unassigned      YES unset  up                    up

Ethernet2/3            unassigned      YES unset  up                    up

...

Loopback1              1.1.1.1         YES NVRAM  up                    up

Vlan1                  192.168.122.82  YES NVRAM  up                    up

Connecting to device 192.168.122.72

Interface                  IP-Address      OK? Method Status                Protocol

Ethernet0/0                192.168.122.72  YES NVRAM  up                    up

Ethernet0/1                unassigned      YES NVRAM  administratively down down

Ethernet0/2                unassigned      YES NVRAM  administratively down down

Ethernet0/3                unassigned      YES NVRAM  administratively down down

Ethernet1/0                unassigned      YES NVRAM  administratively down down

Ethernet1/1                unassigned      YES NVRAM  administratively down down

Ethernet1/2                unassigned      YES NVRAM  administratively down down

Ethernet1/3                unassigned      YES NVRAM  administratively down down

Serial2/0                  unassigned      YES NVRAM  administratively down down

Serial2/1                  unassigned      YES NVRAM  administratively down down

Serial2/2                  unassigned      YES NVRAM  administratively down down

Serial2/3                  unassigned      YES NVRAM  administratively down down

...

Loopback0                  2.2.2.2         YES NVRAM  up                    up

Loopback22                 22.22.22.22     YES NVRAM  administratively down down

Connecting to device 192.168.122.73

Interface              IP-Address      OK? Method Status                Protocol

FastEthernet0/0        192.168.122.73  YES NVRAM  up                    up

FastEthernet1/0        unassigned      YES NVRAM  administratively down down

FastEthernet2/0        unassigned      YES NVRAM  administratively down down

Serial3/0              unassigned      YES NVRAM  administratively down down

Serial3/1              unassigned      YES NVRAM  administratively down down

Serial3/2              unassigned      YES NVRAM  administratively down down

Serial3/3              unassigned      YES NVRAM  administratively down down

---- End get config sequential, elapsed time= 15.61446762084961

root@NetworkAutomation-1:~#

### Section 9 (scaling Netmiko) - Measuring the script execution time in Netmiko parallel deployment

Topology

The following script:

- Connects to remote devices in parallel using SSH (Netmiko)

- Measures the script total execution time

root@NetworkAutomation-1:~# cat >  measure_time_parallel.py

from netmiko import ConnectHandler

from time import time

import threading

ios_sw1 = {

'device_type': 'cisco_ios',

'ip': '192.168.122.82',

'username': 'mikis',

'password': 'cisco'

}

ios_r1 = {

'device_type': 'cisco_ios',

'ip': '192.168.122.72',

'username': 'mikis',

'password': 'cisco'

}

ios_r2 = {

'device_type': 'cisco_ios',

'ip': '192.168.122.73',

'username': 'mikis',

'password': 'cisco'

}

# A list with all devices

all_devices = [ios_sw1, ios_r1, ios_r2]

# Function which connects to a remote device using Netmiko

# Executes a 'show' command and prints the output

def worker(ip):

print("\nConnecting to device " + device["ip"])

net_connect = ConnectHandler(**device)

output = net_connect.send_command("show ip int brief")

print ('\n---- End get config parallel, elapsed time=', time()-starting_time)

return print(output)

# Main program with threading

# The main script starts

threads = []

starting_time = time()

for device in all_devices:

t = threading.Thread(target=worker, args=(device["ip"],))

threads.append(t)

t.start()

Running the script

root@NetworkAutomation-1:~# python3 measure_time_parallel.py

Connecting to device 192.168.122.82

Connecting to device 192.168.122.72

Connecting to device 192.168.122.73

---- End get config parallel, elapsed time= 5.510881662368774

Interface                  IP-Address      OK? Method Status                Protocol

Ethernet0/0                192.168.122.72  YES NVRAM  up                    up

Ethernet0/1                unassigned      YES NVRAM  administratively down down

Ethernet0/2                unassigned      YES NVRAM  administratively down down

Ethernet0/3                unassigned      YES NVRAM  administratively down down

Ethernet1/0                unassigned      YES NVRAM  administratively down down

Ethernet1/1                unassigned      YES NVRAM  administratively down down

Ethernet1/2                unassigned      YES NVRAM  administratively down down

Ethernet1/3                unassigned      YES NVRAM  administratively down down

Serial2/0                  unassigned      YES NVRAM  administratively down down

Serial2/1                  unassigned      YES NVRAM  administratively down down

Serial2/2                  unassigned      YES NVRAM  administratively down down

Serial2/3                  unassigned      YES NVRAM  administratively down down

Serial3/0                  unassigned      YES NVRAM  administratively down down

Serial3/1                  unassigned      YES NVRAM  administratively down down

Serial3/2                  unassigned      YES NVRAM  administratively down down

Serial3/3                  unassigned      YES NVRAM  administratively down down

Loopback0                  2.2.2.2         YES NVRAM  up                    up

Loopback22                 22.22.22.22     YES NVRAM  administratively down down

---- End get config parallel, elapsed time= 5.514587163925171

Interface                  IP-Address      OK? Method Status                Protocol

Ethernet0/0                192.168.122.72  YES NVRAM  up                    up

Ethernet0/1                unassigned      YES NVRAM  administratively down down

Ethernet0/2                unassigned      YES NVRAM  administratively down down

Ethernet0/3                unassigned      YES NVRAM  administratively down down

Ethernet1/0                unassigned      YES NVRAM  administratively down down

Ethernet1/1                unassigned      YES NVRAM  administratively down down

Ethernet1/2                unassigned      YES NVRAM  administratively down down

Ethernet1/3                unassigned      YES NVRAM  administratively down down

Serial2/0                  unassigned      YES NVRAM  administratively down down

Serial2/1                  unassigned      YES NVRAM  administratively down down

Serial2/2                  unassigned      YES NVRAM  administratively down down

Serial2/3                  unassigned      YES NVRAM  administratively down down

Serial3/0                  unassigned      YES NVRAM  administratively down down

Serial3/1                  unassigned      YES NVRAM  administratively down down

Serial3/2                  unassigned      YES NVRAM  administratively down down

Serial3/3                  unassigned      YES NVRAM  administratively down down

Loopback0                  2.2.2.2         YES NVRAM  up                    up

Loopback22                 22.22.22.22     YES NVRAM  administratively down down

---- End get config parallel, elapsed time= 5.747814893722534

Interface              IP-Address      OK? Method Status                Protocol

FastEthernet0/0        192.168.122.73  YES NVRAM  up                    up

FastEthernet1/0        unassigned      YES NVRAM  administratively down down

FastEthernet2/0        unassigned      YES NVRAM  administratively down down

Serial3/0              unassigned      YES NVRAM  administratively down down

Serial3/1              unassigned      YES NVRAM  administratively down down

Serial3/2              unassigned      YES NVRAM  administratively down down

Serial3/3              unassigned      YES NVRAM  administratively down down

root@NetworkAutomation-1:~#

### Lecture 105 - Python3 - new way of working with strings and variables

The following script:

- Converts decimal to hex

root@NetworkAutomation-2:~# cat dec_to_hex.py

dec = int(input ("Give me a decimal number: "))

a = hex(dec)

print ("{} in decimal is {} in hex".format(dec, a))

Running the script

root@NetworkAutomation-2:~# python3 dec_to_hex.py

Give me a decimal number: 10

10 in decimal is 0xa in hex

root@NetworkAutomation-2:~# python3 dec_to_hex.py

Give me a decimal number: 100

100 in decimal is 0x64 in hex

### Lecture 106 - Check built-in methods for Python

Use the dir function:

root@NetworkAutomation-2:~# python3

Python 3.5.2 (default, Nov 23 2017, 16:37:01)

[GCC 5.4.0 20160609] on linux

Type "help", "copyright", "credits" or "license" for more information.

>>>

>>> dir()

['__builtins__', '__doc__', '__loader__', '__name__', '__package__', '__spec__']

>>>

>>>

>>> dir(str)

['__add__', '__class__', '__contains__', '__delattr__', '__dir__', '__doc__', '__eq__', '__format__', '__ge__', '__getattribute__', '__getitem__', '__getnewargs__', '__gt__', '__hash__', '__init__', '__iter__', '__le__', '__len__', '__lt__', '__mod__', '__mul__', '__ne__', '__new__', '__reduce__', '__reduce_ex__', '__repr__', '__rmod__', '__rmul__', '__setattr__', '__sizeof__', '__str__', '__subclasshook__', 'capitalize', 'casefold', 'center', 'count', 'encode', 'endswith', 'expandtabs', 'find', 'format', 'format_map', 'index', 'isalnum', 'isalpha', 'isdecimal', 'isdigit', 'isidentifier', 'islower', 'isnumeric', 'isprintable', 'isspace', 'istitle', 'isupper', 'join', 'ljust', 'lower', 'lstrip', 'maketrans', 'partition', 'replace', 'rfind', 'rindex', 'rjust', 'rpartition', 'rsplit', 'rstrip', 'split', 'splitlines', 'startswith', 'strip', 'swapcase', 'title', 'translate', 'upper', 'zfill']

>>>

>>> a = 'test'

>>> dir(a)

['__add__', '__class__', '__contains__', '__delattr__', '__dir__', '__doc__', '__eq__', '__format__', '__ge__', '__getattribute__', '__getitem__', '__getnewargs__', '__gt__', '__hash__', '__init__', '__iter__', '__le__', '__len__', '__lt__', '__mod__', '__mul__', '__ne__', '__new__', '__reduce__', '__reduce_ex__', '__repr__', '__rmod__', '__rmul__', '__setattr__', '__sizeof__', '__str__', '__subclasshook__', 'capitalize', 'casefold', 'center', 'count', 'encode', 'endswith', 'expandtabs', 'find', 'format', 'format_map', 'index', 'isalnum', 'isalpha', 'isdecimal', 'isdigit', 'isidentifier', 'islower', 'isnumeric', 'isprintable', 'isspace', 'istitle', 'isupper', 'join', 'ljust', 'lower', 'lstrip', 'maketrans', 'partition', 'replace', 'rfind', 'rindex', 'rjust', 'rpartition', 'rsplit', 'rstrip', 'split', 'splitlines', 'startswith', 'strip', 'swapcase', 'title', 'translate', 'upper', 'zfill']

>>>

>>>

>>> a.capitalize()

'Test'

>>>

>>> a.casefold()

'test'

>>>

>>> a.count('t')

2

### Section 14 (telnetlib) - Script 1

The following script:

- Connects via telnet to a device

- The IP of the device is specified manually (the HOST variable)

- The user is prompted for user and password

- The provided credentials are used by the script in the telnet connection

- The script collects 'show version' output from the remote device

root@NetworkAutomation-1:~# cat > show_ver.py

import getpass

import telnetlib

# Ask for username and password

ip = "192.168.122.82"

username = input("Enter your username: ")

password = getpass.getpass()

tn = telnetlib.Telnet(ip)

tn.read_until(b"Username: ")

tn.write(username.encode('ascii') + b"\n")

if password:

tn.read_until(b"Password: ")

tn.write(password.encode('ascii') + b"\n")

tn.write(b"terminal length 0\n")

tn.write(b"show version\n")

tn.write(b"exit\n")

print (tn.read_all().decode('ascii'))

Running the script

root@NetworkAutomation-1:~# python3 show_ver.py

Enter your username: mikis

Password:

SW1#terminal length 0

SW1#show version

Cisco IOS Software, Solaris Software (I86BI_LINUXL2-ADVENTERPRISEK9-M), Experimental Version 15.1(20130726:213425) [dstivers-july26-2013-team_track 104]

Copyright (c) 1986-2013 by Cisco Systems, Inc.

Compiled Fri 26-Jul-13 15:56 by dstivers

ROM: Bootstrap program is Linux

SW1 uptime is 38 minutes

System returned to ROM by reload at 0

System image file is "unix:/opt/gns3/images/IOU/i86bi-linux-l2-adventerprisek9-15.1a.bin"

Last reload reason: Unknown reason

This product contains cryptographic features and is subject to United

States and local country laws governing import, export, transfer and

use. Delivery of Cisco cryptographic products does not imply

third-party authority to import, export, distribute or use encryption.

Importers, exporters, distributors and users are responsible for

compliance with U.S. and local country laws. By using this product you

agree to comply with applicable laws and regulations. If you are unable

to comply with U.S. and local laws, return this product immediately.

A summary of U.S. laws governing Cisco cryptographic products may be found at:

http://www.cisco.com/wwl/export/crypto/tool/stqrg.html

If you require further assistance please contact us by sending email to

export@cisco.com.

Linux Unix (Intel-x86) processor with 189092K bytes of memory.

Processor board ID 2048001

16 Ethernet interfaces

1 Virtual Ethernet interface

16K bytes of NVRAM.

Configuration register is 0x0

SW1#exit

root@NetworkAutomation-1:~#

### Section 14 (telnetlib) - Script 2

The following script:

- Connects via telnet to a device

- The IP of the device is specified manually (the 'ip' variable)

- The user is prompted for user and password

- The provided credentials are used by the script in the telnet connection

- The script collects 'show version' output from the remote device and shows a specific part

root@NetworkAutomation-1:~# cat > show_ver2.py

import getpass

import telnetlib

# Ask for username and password

ip = "192.168.122.82"

username = input("Enter your username: ")

password = getpass.getpass()

# Telnet to the remote device

tn = telnetlib.Telnet(ip)

# Read the remote output until you see 'Username: ' string

# Then enter the provided username and 'Enter' character

tn.read_until(b"Username: ")

tn.write(username.encode('ascii') + b"\n")

# If the password was provided

# Read the remote output until you see 'Password: ' string

# Then enter the provided password and 'Enter' character

if password:

tn.read_until(b"Password: ")

tn.write(password.encode('ascii') + b"\n")

# Send the following commands to the remote device

# And print the output

tn.write(b"terminal length 0\n")

tn.write(b"show version\n")

tn.write(b"exit\n")

getversion = tn.read_all().decode('ascii')

print (getversion[41:157])

Running the script

root@NetworkAutomation-1:~# python3 show_ver2.py

Enter your username: mikis

Password:

Cisco IOS Software, Solaris Software (I86BI_LINUXL2-ADVENTERPRISEK9-M), Experimental Version 15.1(20130726:213425)

### Section 14 (telnetlib) - Script 3 - print length in bytes of specific part of output

The following script:

- Connects via telnet to a device

- The IP of the device is specified manually (the 'ip' variable)

- The user is prompted for user and password

- The provided credentials are used by the script in the telnet connection

- The script collects 'show version' output from the remote device and shows the length in bytes (characters)

root@NetworkAutomation-1:~# cat > show_ver3.py

import getpass

import telnetlib

# Ask for username and password

ip = "192.168.122.82"

username = input("Enter your username: ")

password = getpass.getpass()

# Telnet to the remote device

tn = telnetlib.Telnet(ip)

# Read the remote output until you see 'Username: ' string

# Then enter the provided username and 'Enter' character

tn.read_until(b"Username: ")

tn.write(username.encode('ascii') + b"\n")

# If the password was provided

# Read the remote output until you see 'Password: ' string

# Then enter the provided password and 'Enter' character

if password:

tn.read_until(b"Password: ")

tn.write(password.encode('ascii') + b"\n")

# Send the following commands to the remote device

# And print the output

tn.write(b"terminal length 0\n")

tn.write(b"show version\n")

tn.write(b"exit\n")

print (len(tn.read_all().decode('ascii')))

Running the script

root@NetworkAutomation-1:~# python3 show_ver3.py

Enter your username: mikis

Password:

1533

### Section 14 (telnetlib) - Script 4 - print number of occurrences

The following script:

- Connects via telnet to a device

- The user is prompted for user and password

- The provided credentials are used by the script in the telnet connection

- The script collects 'show ip int brief' output from the remote device and shows the number of occurrences of specific string

root@NetworkAutomation-1:~# cat > show_count.py

import getpass

import telnetlib

# Ask for username and password

ip = "192.168.122.82"

username = input("Enter your username: ")

password = getpass.getpass()

# Telnet to the remote device

tn = telnetlib.Telnet(ip)

# Read the remote output until you see 'Username: ' string

# Then enter the provided username and 'Enter' character

tn.read_until(b"Username: ")

tn.write(username.encode('ascii') + b"\n")

# If the password was provided

# Read the remote output until you see 'Password: ' string

# Then enter the provided password and 'Enter' character

if password:

tn.read_until(b"Password: ")

tn.write(password.encode('ascii') + b"\n")

# Send the following commands to the remote device

# And print the output

tn.write(b"terminal length 0\n")

tn.write(b"show ip int brief\n")

tn.write(b"exit\n")

# Read the whole output, count the 'net' occurrences and print the number

str1 = tn.read_all()

count1 = str1.count(b"net")

print (count1)

Running the script

root@NetworkAutomation-1:~# python3 show_count.py

Enter your username: mikis

Password:

16

### Section 20 - While loop 1

The goal

I want to do this:
root@NetworkAutomation-1:~# python3 whileloop1.py

0

1

2

3

4

The solution

root@NetworkAutomation-1:~# cat > whileloop1.py

counter = 0

while counter < 5:

print (counter)

counter += 1

### Section 20 - While loop 2 - Infinite while with a break

The goal

I want to do this:

root@NetworkAutomation-1:~# python3 whileloop2.py

String to make lowercase [type "q" t quit]: Test

test

String to make lowercase [type "q" t quit]: 1q

1q

String to make lowercase [type "q" t quit]: q

Loop broken!

root@NetworkAutomation-1:~#

The solution

root@NetworkAutomation-1:~# cat > whileloop2.py

while True:

textin = input('String to make lowercase [type "q" t quit]: ')

if textin == 'q':

break

print (textin.lower())

print ('Loop broken!')

### Section 20 - While loop 3 - Infinite while with continue

The goal

I want to do this:

root@NetworkAutomation-1:~# python3 whileloop3.py

Enter an integer value [type 'q' to quite]: 1

The number 1 is odd. Its square is: 1

Enter an integer value [type 'q' to quite]: 2

This was an Even number, the loop continues

Enter an integer value [type 'q' to quite]: 3

The number 3 is odd. Its square is: 9

Enter an integer value [type 'q' to quite]: 4

This was an Even number, the loop continues

Enter an integer value [type 'q' to quite]: 5

The number 5 is odd. Its square is: 25

Enter an integer value [type 'q' to quite]: q

root@NetworkAutomation-1:~#

The solution

root@NetworkAutomation-1:~# cat > whileloop3.py

while True:

textin = input("Enter an integer value [type 'q' to quite]: ")

if textin == 'q':

break

number = int(textin)

# If the number is even then

# from 'continue' will go to the beginning of While loop

if number % 2 == 0:

print ("This was an Even number, the loop continues")

continue

print ('The number', number, 'is odd. Its square is:', number*number)

### Section 20 - While loop 4 - Infinite while with a list

The goal

I want to do this:

root@NetworkAutomation-1:~# python3 whileloop4.py

0 R1

1 R2

2 R3

3 R4

The solution

root@NetworkAutomation-1:~# cat > whileloop4.py

routers = ['R1', 'R2', 'R3', 'R4']

place_in_list = 0

while place_in_list < len(routers):

print (str(place_in_list) + " " + routers[place_in_list])

place_in_list += 1

### Section 20 - While loop 5 - Infinite while with else

The goal

I want to do this:

root@NetworkAutomation-1:~# python3 whileloop5.py

0 R1

1 R2

2 R3

Found router R3 at position: 2

The solution

root@NetworkAutomation-1:~# cat > whileloop5.py

routers = ['R1', 'R2', 'R3', 'R4']

place_in_list = 0

while place_in_list < len(routers):

print(str(place_in_list) + " " + routers[place_in_list])

if routers[place_in_list] == 'R3':

print("Found router R3 at position: %d" % place_in_list)

break

place_in_list += 1

else:

print("Router 'R3' was not found")

### Section 21 - For loop 1a

The goal

I want to do this:

root@NetworkAutomation-1:~# python3 forloop1.py

Vendor is: Cisco

Vendor is: Arista

Vendor is: PAN

Vendor is: Checkpoint

Vendor is: Fortinet

Vendor is: Juniper

root@NetworkAutomation-1:~#

The solution

root@NetworkAutomation-1:~# cat > forloop1.py

vendors = ['Cisco', 'Arista', 'PAN', 'Checkpoint', 'Fortinet', 'Juniper']

for vendor in vendors:

print ("Vendor is:", vendor)

### Section 21 - For loop 1b

The goal

I want to do this:

root@NetworkAutomation-1:~# python3 forloop1b.py

R

o

u

t

e

r

s

a

n

d

S

w

i

t

c

h

e

s

root@NetworkAutomation-1:~#

The solution

root@NetworkAutomation-1:~# cat > forloop1b.py

phrase = "Routers and Switches"

for i in phrase:

print(i)

### Section 21 - For loop 2 - 'for' loop with Dictionary - print Keys

Consider this Python dictionary:

SW1 = {

'device_type': 'cisco_ios',

'ip': '192.168.122.72',

'username': 'mikis',

'password': 'cisco'

}

The goal

I want to print its keys e.g.

root@NetworkAutomation-1:~# python3 forloop2.py

ip

password

username

device_type

The solution

root@NetworkAutomation-1:~# cat > forloop2.py

SW1 = {

'device_type': 'cisco_ios',

'ip': '192.168.122.72',

'username': 'mikis',

'password': 'cisco'

}

for key in SW1:

print(key)

### Section 21 - For loop 2b - 'for' loop with Dictionary - print Values Way 1

The goal

I have a Dictionary and I want to print its values e.g.

root@NetworkAutomation-1:~# python3 forloop2b.py

192.168.122.72

cisco

mikis

cisco_ios

The solution

root@NetworkAutomation-1:~# cat > forloop2b.py

SW1 = {

'device_type': 'cisco_ios',

'ip': '192.168.122.72',

'username': 'mikis',

'password': 'cisco'

}

for key in SW1:

print(SW1[key])

### Section 21 - For loop 2b - 'for' loop with Dictionary - print Values Way 2

The goal

I have a Dictionary and I want to print its values e.g.

root@NetworkAutomation-1:~# python3 forloop2b.py

192.168.122.72

cisco

mikis

cisco_ios

The solution

root@NetworkAutomation-1:~# cat > forloop2c.py

SW1 = {

'device_type': 'cisco_ios',

'ip': '192.168.122.72',

'username': 'mikis',

'password': 'cisco'

}

# Note that if I put something different than 'values' will not work

for value in SW1.values():

print(value)

### Section 21 - For loop 2b - 'for' loop with Dictionary - print Keys and Values

The goal

I have a Dictionary and I want to print its keys and values e.g.

root@NetworkAutomation-1:~# python3 forloop2d.py

('username', 'mikis')

('device_type', 'cisco_ios')

('password', 'cisco')

('ip', '192.168.122.72')

The solution

root@NetworkAutomation-1:~# cat > forloop2d.py

SW1 = {

'device_type': 'cisco_ios',

'ip': '192.168.122.72',

'username': 'mikis',

'password': 'cisco'

}

# Note that if I put something different than 'items' will not work

for item in SW1.items():

print(item)

### Section 21 - For loop 3 - 'for' loop with Lists and If

The goal

I have 2 lists and I want to print their differences e.g.

root@NetworkAutomation-1:~# python3 forloop3.py

Device mikis_R1 is an approved one!

Device mikis_R2 is an approved one!

Device bad_R1 is not in list of approved devices

Device mikis_R3 is an approved one!

Device mikis_R4 is an approved one!

Device bad_R2 is not in list of approved devices

The solution

root@NetworkAutomation-1:~# cat > forloop3.py

all_devices = ['mikis_R1', 'mikis_R2', 'bad_R1', 'mikis_R3', 'mikis_R4', 'bad_R2']

approved_devices = ['mikis_R1', 'mikis_R2', 'mikis_R3', 'mikis_R4']

# Look all items in list 1 (all_devices)

for device in all_devices:

# If an itemt is not in list 2 (approved) devices, print the item and a message

if device not in approved_devices:

print("Device", device, "is not in list of approved devices")

else:

print("Device", device, "is an approved one!")

### Section 22 - Ranges 1 - SSH to a device and create multiple loopbacks

The goal

I have a remote device

I want to connect using SSH and create multiple loopbacks

The user specifies the amount interactively

The solution

root@NetworkAutomation-1:~# cat > multiple_loopbacks2.py

# The following script uses Netmiko to connect to a device

# Then uses a 'for' loop with Range to create loopbacks

# It assigns them IPs from range '192.168.x.x/32'

from netmiko import ConnectHandler

# The remote device

remote_device1 = {

'device_type': 'cisco_ios',

'ip': '192.168.122.73',

'username': 'mikis',

'password': 'cisco'

}

# Get user input for amount of loopbacks

num_of_loopbacks = input("How many loopbacks do you want: ")

# Use SSH (Netmiko) to connect to the remote device

ssh = ConnectHandler(**remote_device1)

for i in range (0,int(num_of_loopbacks)):

print("Creating loopback" + str(i))

loopback_creation = ['interface loopback ' + str(i), 'ip address ' \

+ '192.168.0.' + str(i+1) + ' 255.255.255.255']

output = ssh.send_config_set(loopback_creation)

print(output)

Running the script

root@NetworkAutomation-1:~# python3 multiple_loopbacks2.py

How many loopbacks do you want: 3

Creating loopback0

config term

Enter configuration commands, one per line.  End with CNTL/Z.

R2(config)#interface loopback 0

R2(config-if)#ip address 192.168.0.1 255.255.255.255

R2(config-if)#end

R2#

Creating loopback1

config term

Enter configuration commands, one per line.  End with CNTL/Z.

R2(config)#interface loopback 1

R2(config-if)#ip address 192.168.0.2 255.255.255.255

R2(config-if)#end

R2#

Creating loopback2

config term

Enter configuration commands, one per line.  End with CNTL/Z.

R2(config)#interface loopback 2

R2(config-if)#ip address 192.168.0.3 255.255.255.255

R2(config-if)#end

R2#

root@NetworkAutomation-1:~#

The result

R2#show ip int br | e un

Interface              IP-Address      OK? Method Status                Protocol

FastEthernet0/0        192.168.122.73  YES NVRAM  up                    up

Loopback0              192.168.0.1     YES manual up                    up

Loopback1              192.168.0.2     YES manual up                    up

Loopback2              192.168.0.3     YES manual up                    up

### Section 22 - Ranges 2 - Create multiple loopbacks (<254) in a file

The goal

The user specifies the amount interactively. The loopbacks are written to a file

The solution

root@NetworkAutomation-1:~# cat > multiple_loopbacks3.py

# The following script:

# Asks the user for number of loopbacks

# creates the loopbacks to a file

# It assigns them IPs from range '192.168.x.x/32'

# Create a new file on the local directory

target = open('file.txt', 'a')

# Get user input for amount of loopbacks

num_of_loopbacks = input("How many loopbacks do you want: ")

# 'for' loop which writes the loopbacks to a file

for i in range (0,int(num_of_loopbacks)):

print("Creating loopback" + str(i))

loopback_creation = 'interface loopback ' + str(i) + '\n' + 'ip address ' + '192.168.0.' \

+ str(i+1) + ' 255.255.255.255'

target.write(loopback_creation)

target.write("\n")

#print(output)

print ("The loopbacks were created in file 'file.txt'!")

target.close()

Running the script

root@NetworkAutomation-1:~# python3 multiple_loopbacks3.py

How many loopbacks do you want: 5

Creating loopback0

Creating loopback1

Creating loopback2

Creating loopback3

Creating loopback4

The loopbacks were created in file 'file.txt'!

The result

root@NetworkAutomation-1:~# cat file.txt

interface loopback 0

ip address 192.168.0.1 255.255.255.255

interface loopback 1

ip address 192.168.0.2 255.255.255.255

interface loopback 2

ip address 192.168.0.3 255.255.255.255

interface loopback 3

ip address 192.168.0.4 255.255.255.255

interface loopback 4

ip address 192.168.0.5 255.255.255.255

root@NetworkAutomation-1:~#

### Section 22 - Ranges 2b - Create multiple loopbacks in a file

The goal

The user specifies the amount interactively. The loopbacks are written to a file

If the loopbacks are > 254 then the third octet increases by 1

The solution

root@NetworkAutomation-1:~# cat > multiple_loopbacks4.py

# The following script:

# Asks the user for number of loopbacks

# creates the loopbacks to a file

# It assigns them IPs from range '192.168.x.x/32'

# If # of loopbacks > 254 then the 3rd octet increases by '1'

# Create a new file on the local directory

target = open('file.txt', 'a')

# Get user input for amount of loopbacks

num_of_loopbacks = input("How many loopbacks do you want: ")

# 'for' loop which writes the loopbacks to a file

octet3 = 0

for i in range (1,int(num_of_loopbacks)):

mod = i % 254

if mod == 0:

print("Creating loopback" + str(i))

loopback_creation = 'interface loopback ' + str(i) + '\n' + 'ip address ' + '192.168.' \

+ str(octet3) + '.254' + ' 255.255.255.255'

target.write(loopback_creation)

target.write("\n")

octet3 += 1

else:

print("Creating loopback" + str(i))

loopback_creation = 'interface loopback ' + str(i) + '\n' + 'ip address ' + '192.168.' \

+ str(octet3) + '.' + str(mod) + ' 255.255.255.255'

target.write(loopback_creation)

target.write("\n")

print ("The loopbacks were created in file 'file.txt'!")

target.close()

Running the script

root@NetworkAutomation-1:~# python3 multiple_loopbacks4.py

How many loopbacks do you want: 500

Creating loopback1

Creating loopback2

Creating loopback3

Creating loopback4

Creating loopback5

Creating loopback6

...

Creating loopback497

Creating loopback498

Creating loopback499

The loopbacks were created in file 'file.txt'!

The result

root@NetworkAutomation-1:~# cat file.txt

interface loopback 1

ip address 192.168.0.1 255.255.255.255

interface loopback 2

ip address 192.168.0.2 255.255.255.255

interface loopback 3

ip address 192.168.0.3 255.255.255.255

interface loopback 4

...

interface loopback 254

ip address 192.168.0.254 255.255.255.255

interface loopback 255

ip address 192.168.1.1 255.255.255.255

interface loopback 256

ip address 192.168.1.2 255.255.255.255